# # $ K @ About the IBM ServeRAID Configuration Program

You can use the IBM®Copyright_and_Trademarks_1 Copyright_and_Trademarks_1Copyright_and_Trademarks_1 ServeRAIDÔCopyright_and_Trademarks_1 Configuration Program to do the following:

- View the current disk-array configuration

- Change or delete existing arrays

- Create new disk arrays

- Perform many other RAID system configuration and maintenance tasks.

You must use the ServeRAID Configuration Program to configure your disk arrays before you partition your hard disk drives and install your operating system.  After you configure your disk arrays, you can install your operating system.

When you connect several hard disks together and configure the RAID controller to access them in a particular pattern, you create a disk array.  The ServeRAID controller currently supports up to eight independent arrays.

Disk arrays are used to improve security, performance, and reliability.  The amount of improvement depends on the application programs that you run on the server and the RAID levels that you assign to the logical drives in your arrays.  The ServeRAID controller supports RAID levels 0RAID_Level_0>main, 1RAID_Level_1>main, Enhanced 1RAID_Enhanced_Level_1>main, 5RAID_Level_5>main, and Enhanced 5RAID_Level_5E>main.

If you have one or more ServeRAID controllers that are not configured when the configuration program starts, the program starts in Configuration ModeConfiguration_Mode and the Configure the ServeRAID ControllerConfigure_Adapter_Screen>main screen appears.

Note:  You can install multiple ServeRAID controllers in your system, but you must configure each controller separately.

If all of your ServeRAID controllers are already configured when the configuration program starts, the program start in Information ModeInformation_Mode and the System InformationCurrent_Status_Screen_Pi>main screen appears.

See Also

Finding Information on the World Wide WebFinding_Information_on_the_World_Wide_Web>main

What's New in the IBM ServeRAID Configuration ProgramWhat_s_New>gloss

Basic Steps in Configuring a ServeRAID ControllerBasic_Steps_in_Configuring_a_ServeRAID_Controller>main

System Information ScreenCurrent_Status_Screen_Pi>main

Configure the ServeRAID Controller ScreenConfigure_Adapter_Screen>main

Custom Configuration Physical Drive ScreenCustom_Configuration_Screen_B>main

Custom Configuration Logical Drive ScreenCustom_Configuration_Logical_Drive_Screen_C>main

Configuration Summary ScreenExpress_Configuration_Screen_D

Event ViewerEvent_Viewer>detail2

# # $ K @ System Information Screen

If all of your ServeRAID controllers are already configured when the configuration program starts, or if you have completed configuring your controllers, this screen appears.

If you have specified and applied any configuration for the ServeRAID controller, then depending on your controller and firmware, a progress indicator might appear in the upper right-hand screen.  The indicator reports the progress of logical drive synchronization.  If your controller does not support automatic synchronization, when you exit the configuration program a screen appears requesting that you synchronize the logical drives.

The Tree View on the left-hand screen represents the RAID subsystem in your server.  You can expand and collapse the tree to view different components of your server configuration.  When you select a specific component from the tree, the information associated with that component appears on the right-hand screen.

See Also

Understanding System Information IconsUnderstanding_Tree_Component_Icons>icon

Using the System Information ScreenCurrent_Status_Details_View>detail2

Event ViewerEvent_Viewer>detail2

# # $ K @ Using the System Information Screen

- Expand the Tree View of your server in the left-hand screen.

- Select a specific component in the tree.  The associated information for that component appears on the right-hand screen.

See Also

Understanding System Information IconsUnderstanding_Tree_Component_Icons>icon

# # $ K @ Understanding System Information Icons

You can view the following components in the System Information tree view:

{bmlt s_server1.BMP}   ServerServer_System_Information>detail2

{bmct s_adapt1.BMP}   ControllerAdapter_System_Information>detail2

{bmlt s_arrays1.BMP}   ArraysArray_Information>detail2

{bmlt s_logdrs1.BMP}   Logical DrivesLogical_Drive_Information>detail2

{bmlt s_phdrs1.BMP}   Physical DrivesPhysical_Drives_Information>detail2

{bmlt s_hsps1.BMP}   Hot-Spare Drives Hot_Spare_Drive_Information>detail2

{bmlt s_array1.BMP}   Array ?Array_Query_Information>detail2

{bmlt s_logdrs1.BMP}   Logical Drives in Array ?Logical_Drives_in_Array_Query_Information>detail2

{bmlt s_phdrs1.BMP}   Physical Drives In Array ?Physical_Drives_in_Arrays_Query_Information>detail2

{bmlt s_logdr1.BMP}   Logical Drive ?Logical_Drive_Query_Information>detail2

{bmlt s_pdrdy.BMP}   Physical Drive ?Physical_Drive_Information>detail2

{bmlt s_scsi1.BMP}   Channel ?Channel_Information>detail2

# $ K @ Configure the ServeRAID Controller Screen

The ServeRAID controllers support up to eight (8) independent arrays.  You must use the Express or the Custom configuration mode to configure your disk arrays before you partition your hard disk drives and install your operating system.

The Configure Controller screen appears when:

- One or more controllers have available Ready drives and there are no logical drives defined when the Configuration Program starts.

- You select Configure Controller from within the information view

Tip:  On this screen, you can select only controllers, but you can expand and collapse the entire tree.

Express configuration provides a quick and easy path for you to automatically configure your ServeRAID controller.  This choice creates the most efficient ServeRAID configuration based on the number and capacity of the Ready drives available in your system.  If four or more Ready drives of the same capacity are available, this choice also will define a hot-spare drive.  A hot-spare drive is a physical drive that is defined for automatic use when a similar drive fails.

The Express configuration choice groups up to 16 Ready drives of the same capacity into one disk array and defines one logical drive for each array.  This choice defines the size of the logical drive, based on the amount of free space available; and, it assigns the highest RAID level possible, based on the number of physical drives available.

The Custom configuration path allows you to manually configure your ServeRAID subsystem.  Using this path, you can select the drives that you want to include in each array, define the number and size of the logical drives for each array, and select the drives that you want to use as hot spares.

See Also

Express Configuration ExampleExpress_Configuration_Examples>detail2

Understanding the Tree View IconsTree_View_Icons>icon

Using the Configure Controller ScreenConfigure_Adapter_Detail_View>detail2

Event ViewerEvent_Viewer>detail2

# # $ K @ Express Configuration Example

If your server contains one 1024 MBMB Ready drive, two 2150 MB Ready drives, and four 4300 MB Ready drives, Express Configuration will create three arrays and one hot-spare drive as follows:

Array A:  The total capacity of this array is 1024 MB (1 x 1024 MB) and it contains one, 1024 MB RAID level-0 logical drive.

Array B:  The total capacity of this array is 4300 MB (2 x 2150 MB) and it contains one, 2150 MB RAID level-1 logical drive.

Array C:  The total capacity of this array is 12900 MB (3 x 4300 MB) and it contains one, 8600 MB RAID level-5 logical drive.

Hot Spare:  Express Configuration defines one of the four 4300 MB drives as a hot-spare drive, as follows:

- When there are four or more Ready drives of the same capacity, Express Configuration groups three of the drives into one array (as in Array C) and defines one of the drives as a hot spare.

- A hot-spare drive must be of equal or greater capacity than the drive that it is intended to replace.  In this configuration, the 4300 MB drive can replace any failed drives in Array B or Array C.

# $ K @ Using the Configure Controller Screen

- Select a controller {bmct s_adapt1.BMP} from the tree view that you want to configure.

Tip:  You can select only controllers, but you can expand and collapse the entire tree.

- Select either the Express or the Custom configuration mode; then, click on Next.

Note:  If you select the Express mode, the configuration program automatically:

- Creates one or more disk arrays, based on the number of Ready physical drives in your system.

- Defines a logical drive for each array.

- If four or more Ready physical drives are available, defines a hot-spare drive.

If you select the Custom mode, the configuration program will walk you through the screens needed to:

- Select the physical drives you want to include in your disk arrays.

- Define the logical drives for your arrays.

- Define the hot-spare drives for your arrays.

- In Express mode, after you review the Configuration Summary,Express_Configuration_Summary_View>detail2 select Apply and the ServeRAID Configuration Program will configure the controller and initialize and synchronize the logical drive.

In Custom mode, you can begin creating arrays and hot-spare drives by proceeding to the Custom Configuration Physical Drive ScreenCustom_Configuration_Detail_Tree_View>detail2.

See Also

Understanding the Tree View IconsTree_View_Icons>icon

Express Configuration ExampleExpress_Configuration_Examples>icon

Using the Custom Configuration Physical Drive ScreenCustom_Configuration_Detail_Tree_View>detail2

Using the Summary ScreenExpress_Configuration_Summary_View>detail2

# # $ K @ Custom Configuration Physical Drive Screen

The tree view of your arrays (that is, the right-hand screen) shows you the IDs and sizes (in megabytes) of the arrays.  This tree shows the following:

- The arrays that will be defined when you complete the configuration.

- The hot-spare drives that will be defined when you complete the configuration.

- The size of all the arrays.

- The size of all the physical drives.

There are two types of items in the right-hand tree:  new items and other items.  You can change new items if they are not in the configuration yet.  You cannot change or delete other items.

See Also

Understanding Custom Configuration Physical Drive IconsUnderstanding_Custom_Configuration_Physical_Drive_Icons>icon

Using the Custom Configuration Physical Drive ScreenCustom_Configuration_Detail_Tree_View>detail2

Event ViewerEvent_Viewer>detail2

# # $ K @ Using the Custom Configuration Physical Drive Screen

- Create arrays by dragging physical drives from the left-hand tree to an icon in the right-hand tree labeled either:

- Add New Array

- New Array followed by a letter (such as, New Array B).

The icon in the left-hand tree will move with the drive and appear in the right-hand tree.

Tip:  You also can drag an entire channel {bmct s_scsi1.BMP}, if you want to move all the Ready drives in that channel.

If you want to remove a physical drive {bmct s_pdrdy.BMP} from a new array, drag it back to the left-hand tree.

- Create hot-spare drives by dragging physical drives from the left-hand tree to the icon labeled Hot-Spare Drives in the right-hand tree.  The icon in the left-hand tree will change to the hot-spare icon {bmct s_pdhsp.BMP} and the drive will appear in the Hot-Spare Drives branch in the right-hand tree.

Tip:  If you want to change a New Hot Spare back into a Ready drive, drag it back to the left-hand tree.

- After you define all your arrays and hot-spare drives, click on Next to proceed to the Custom Configuration Logical Drive ScreenCustom_Configuration_Logical_Drive_Details_View>detail2.  You might want to verify that there are no remaining Ready drives

See Also

Understanding Custom Configuration Physical Drive IconsUnderstanding_Custom_Configuration_Physical_Drive_Icons>icon

Notes about the Custom Configuration Physical Drive ScreenUsing_this_screen>detail2

Using the Custom Configuration Logical Drive ScreenCustom_Configuration_Logical_Drive_Details_View>detail2

# # $ K @ Understanding Custom Configuration Physical Drive Icons

The tree view of the controller (that is, the left-hand screen) uses the following icons:

{bmct s_adapt1.BMP}	Controller

{bmct s_phdrs1.BMP}	Physical Drives

{bmct s_pdrdy.BMP}	Ready Drive

{bmct s_pdhsp.BMP}	New Hot-Spare Drive

{bmct s_pdonl.BMP}	New Online Drive

{bmct s_scsi1.BMP}	SCSI Channel

The tree view of the arrays (that is, the right-hand screen) uses the following icons:

{bmct s_adapt1.BMP}	Controller

{bmct s_arrays1.BMP}	Arrays (plural)

{bmct s_array1.BMP}	New Array (singular)

{bmct s_array1.BMP}	Add New Array (singular)

{bmct s_hsps1.BMP}	Hot-Spare Drives (plural)

{bmct s_pdhsp.BMP}	New Hot-Spare Drive (singular)

{bmct s_pdonl.BMP}	New Online Drive

# # $ K @ Notes about the Custom Configuration Physical Drive Screen

- Instead of dragging and dropping, you also can use the right mouse button to click on a physical drive or any other tree item and see a list of available actions for that item.

- All available actions for the selected item are also in the Actions menu.

- You can change only new items.  If you want to change an existing array, you must delete the array.  To delete the array:

- Click on Cancel out of the Custom Configuration screen.

- Click on Yes in the confirmation dialog box.

- Select the array you want to change.

- Select Actions; then, select Delete array.

- Click on Yes in the confirmation dialog box.

- Select the controller from the tree and click on Configure Controller.

- If you change your mind after selecting the drives for an array, you can delete the array; then, begin again.

- When the stripe-unit sizeStripe_Unit_Size is 8 K (the default setting) or 16 K (a manual setting), the maximum number of physical drives in an array is 16.  When the stripe-unit size is 32K or 64K (both are manual settings), the maximum number of physical drives in an array is eight.  You can change the stripe-unit size from the information view.

- Hard disk (physical) drive capacitiesHard_Disk_Drives_Capacities>detail2 influence the way you create arrays.  Physical drives in the array can be of different capacities, but the ServeRAID controller treats them as if they all have the capacity of the smallest disk drive.

# $ K @ Custom Configuration Logical Drive Screen

ServeRAID controllers support up to eight logical drivesLogical_Drives>detail2.  Each array can be one logical drive, or you can divide it into multiple logical drives, as long as the total number of logical drives for all of the arrays defined for the current controller does not exceed eight.

If you do not define a logical drive for every array, the ServeRAID Configuration Program will define a logical drive with the largest capacity and highest RAID level possible.

Note:  Refer to the documentation provided with your operating system for information about the recommended logical-drive size.

Limitations to think about:

- You can have up to eight (8) arrays on a controller.

- You can have up to eight (8) logical drives on a controller.

See Also

Understanding Custom Configuration Logical Drive IconsUnderstanding_Custom_Configuration_Logical_Drive_Icons>icon

Using the Custom Configuration Logical Drive ScreenCustom_Configuration_Logical_Drive_Details_View>detail2

Event ViewerEvent_Viewer>detail2

# # $ K @ Using the Custom Configuration Logical Drive Screen

- Select the array from the tree for which you want to create logical drives.  The pie chart represents the selected array.  The amount of free spaceFree_Space and total space (MBMB) available in the array appears to the right of the pie chart.

Note:  You can define up to eight logical drives for your array.  If you plan to use the logical-drive migration (LDM) procedure, do not define more than seven logical drives.  The LDM procedure requires one free logical drive.

- Select the RAID levelMore_About_Selecting_the_RAID_Level>detail2 that you want to assign to the logical drive from the RAID Level drop-down list box.  You can select RAID Level-0RAID_Level_0>main, RAID Level-1RAID_Level_1>main, RAID Level-5RAID_Level_5>main, or RAID Level-5E.RAID_Level_5E>main

Tip:  Typically, the first logical drive that you define will be your startup (boot) drive.

- Select a size for the logical driveMore_About_Selecting_the_Logical_Drive_Size>detail2.  To select the size (that is, the size in MB that will be used to store data), do either of the following:

- Use the edit field to define the size.

- Use the slider to define the total size as a percent of the total array capacity.

Tip:  If you already defined other logical drives for this array, the pie chart shows the existing logical drive sizes and the amount of free space available.

- After you define all of your logical drives, click on the Next button and proceed to the Summary ScreenExpress_Configuration_Summary_View>main.

See Also

Understanding Custom Configuration Logical Drive IconsUnderstanding_Custom_Configuration_Logical_Drive_Icons>icon

More About Selecting the RAID LevelMore_About_Selecting_the_RAID_Level>(w95sec)

More About Selecting the Logical Drive SizeMore_About_Selecting_the_Logical_Drive_Size>(w95sec)

Logical Drive ExamplesLogical_Drive_Examples>(w95sec)

Using the Summary ScreenExpress_Configuration_Summary_View>detail2

# # $ K @ More About Selecting the RAID Level

- The ServeRAID Configuration Program automatically assigns RAID level-0RAID_Level_0>main to any logical drives defined in an array containing only one hard disk drive.

- If you have two hard disk drives in the array, you can select either RAID level-0RAID_Level_0>main or RAID level-1RAID_Level_1>main.  The default is RAID level-1.

- If you have three or more hard disk drives, you can select RAID Level-0, RAID Level-1, or RAID level-5RAID_Level_5>main.  If you select RAID Level-1, you automatically get RAID level-1 EnhancedRAID_Enhanced_Level_1>main.

- If you plan to use the Change RAID Level feature of the LDM, you must assign the same RAID level to all of the logical drives in the array.

- If you select RAID level-5 EnhancedRAID_Level_5E>main, you can have only one logical drive in an array.

# # $ K @ More About Selecting the Logical Drive Size

- If you change the RAID level of a logical drive, the total size (data plus parity) of the logical drive might change.

- If you did not use all of the available free space, you can create another logical drive.  Select the Add Logical Drive note tab to do so.  You can assign the same or a different RAID level to additional logical drives.

- If you do not want a specific logical drive, select it and click on the Delete button.  If the logical drive is in an array that is not currently selected, select the logical drive from the left-hand tree.  If the drive is on the currently selected array, you can select the drive by clicking its tab.

# # $ K @ Understanding Custom Configuration Logical Drive Icons

The tree view of the controller (that is, the left-hand screen) uses the following icons:

{bmct s_adapt1.BMP}	Controller

{bmct s_arrays1.BMP}	Arrays (plural)

{bmct s_array1.BMP}	New Array (singular)

{bmct s_logdr1.BMP}	New Logical Drive

# # $ K @ Logical Drive Examples

Example 1:

The array consists of three 1 GBGB>gloss drives.  You assign RAID level 0RAID_Level_0>main, which uses all the drives in the array with no redundant or parity storage; then, you type 1000.  The logical drive will actually only use 999 MBMB>gloss because it is the number closest to and lower than 1000 that is divisible by 3.

Example 2:

The array consists of three 1 GB drives.  You assign RAID level 1RAID_Enhanced_Level_1>main, which provides disk mirroring and stripes data across all drives in the array.  Because the data is mirrored, the capacity of the logical drive is 50% of the physical capacity of the hard disk drives grouped in the array.  If you type 1000, the physical capacity used is 2000 MB, twice that of the logical drive size.  The number that is closest to and lower than 2000 that is divisible by 3 is 1998, which will be the actual space used.  The logical drive size is 999 MB, which is 50% of the physical capacity.

Example 3:

The array consists of three 1 GB drives and you assign RAID level 5RAID_Level_5>main. Data is striped across all three drives in the array, but the space equivalent to that of one drive is used for redundant storage.  Therefore, if you type 1000, the number 1000 remains in the Size (MB) field because it is divisible by 2 (drives), which is the space available for data.  The physical capacity used is 1500 MB.

# # $ K @ Configuration Summary Screen

The Configuration Summary screen lists all the changes that the ServeRAID Configuration Program will apply to the configuration:

- All new arrays with their associated logical drives.

- New hot-spare drives.

See Also

Understanding the Tree View IconsTree_View_Icons>icon

Using the Summary ScreenExpress_Configuration_Summary_View>detail2

Event ViewerEvent_Viewer>detail2

# # $ K @ Using the Summary Screen

- Review the configuration in the right-hand screen.  It describes the new arrays, logical drives, and physical drives that will be defined when you click on the Apply button.  There are also notes and attentionsAbout_Notes_and_Attentions>detail2 regarding the configuration.

You can expand the tree for another view of the configuration.

- If you want to modify the configuration:

- To modify arrays, click on {bmct l_arrays1.BMP}Custom_Configuration_Screen_B>detail2.

- To modify logical drives, click on {bmct l_logdrs1.BMP}Custom_Configuration_Logical_Drive_Screen_C>detail2.

- Click on the Apply button when you are satisfied with the configuration.

- Click on Yes when the confirm pop-up window appears.  If your firmware supports it, the ServeRAID Configuration Program will start the synchronization process automatically. Your logical drives will always be initialized automatically.

Note:  If you have disabled automatic initializationViewOptions, the ServeRAID Configuration Program will not initialize the logical drives automatically.

If you have not already done so, install your operating system.

See Also

Understanding the Tree View IconsTree_View_Icons>icon

About Notes and AttentionsAbout_Notes_and_Attentions>detail2

# # $ K @ Notes and Attentions

In the Summary screen, Notes appear in blue and Attentions appear in red.

Note:   Array storage space still available.

Explanation: You can create another logical drive.

Note:  Physical drives contain unusable capacity.

Explanation: You could redefine your arrays with the same size drive capacities to avoid wasting space.

Note:  There is(are) n Ready drive(s) still available. (where n is a number)

Explanation: You can create another array for this drive.

Attention:  This hot-spare drive will not work for any defined array.

Explanation: This hot-spare drive is not big enough to replace a failed drive from any array.

Attention:  No hot-spare drive is available for array L.  (where L is a letter)

Explanation: If a drive fails in this array, the system cannot rebuild it automatically.  You must do so manually.

# $ K @ Event Viewer

The Event Viewer is the region along the bottom of your screen.  The Event Viewer displays information, attention, and error events.  Every event has a date and time stamp and an iconEvent_Log_Icons.

# $ K @ Configure Clustering for ServeRAID

This feature lets you configure RAID so that you can clusterCluster computers.  You will need clustering hardware and software to configure a cluster.

# Click on the Back button to return to the proceeding step in your configuration path.

# Click on the Next button to continue to the next screen.

# Click on the Cancel button to stop configuration of the controller without saving any changes.

# Contains basic program functions, such as Save Configuration Event Log, Clear Configuration Event Log, and Exit.

# Contains different icon viewing selections and the View Options dialog box.

# Contains different choices available for the currently selected object.

# Contains help and program information.

# Saves configuration event information (such as the start time and completion time for a rebuild operation) to a specified file.

# Erases the configuration event information stored in the Event Log.

# Select Exit to leave the IBM ServeRAID Configuration Program.

# Displays large icons in the Main Tree on the left and the Main Panel on the right.

# Displays small icons in the Main Tree on the left and the Main Panel on the right.  This is the default setting.

# Changes the Initialization and Synchronization Mode.  You can choose Basic (the default setting) or Advanced.

# {bmct tbadapt.BMP}

Switches from the Information Mode to the Configuration Mode.  If there are Ready drives and logical-drive slots still available for the ServeRAID controller, you can select this choice to configure a controller.

# Deletes all arrays and resets all the controller parameters to factory defaults.

# When enabled (the default), the controller transfers data from a disk to its local cache in increments equal to the stripe-unit size.  When disabled, the controller transfers data from a disk to its local cache in increments equal to the system I/O request size, without reading ahead to the end of the stripe.  (Information Mode only.)

# {bmct tbscan.BMP}

Scans the individual channel, or all channels on the ServeRAID controller for new or removed Ready drives.  (Information Mode only.)

# Sets the priority for Rebuild operations to High (default), Medium, or Low.  (Information Mode only.)

- When the rebuild priority is set to High, the rebuild I/O request gets highest priority in the execution order.

- When the rebuild request is set from High to Medium in a heavily loaded system, it can increase the disk rebuild time, but provide better system performance.

- When the rebuild request is set from High or Medium to Low in a moderate to heavily loaded system, it can increase the disk rebuild time, but provide better system performance.

# Copies the configuration information stored on the hard disk drives to the ServeRAID controller.  Use this function to import previously configured drives from another system.  (Information Mode only.)

# Sets the size of the chunk of data the controller reads from each drive at a time.  You can select 8 K, 16 K, 32 K, or 64 K.  When you select 8 K (default) or 16 K, the maximum number of physical drives supported in an array is 16.  When you select 32 K (default) or 64 K, the maximum number of physical drives supported in an array is 8.  (Information Mode only.)

To maximize overall performance, choose a size that is close to the size of the system I/O request.

Attention:  After you configure an array and store data on the logical drives, you cannot change the stripe-unit size without destroying data in the logical drives.

# If a defined array has free space with no dead drives, and logical-drive slots are available, create a logical drive in the selected array.

# {bmct tbha.BMP}

Defines the ServeRAID controller for use in a dual-node, high-availability, shared-disk cluster environment.  (Information Mode only.)

# {bmct tbarray.BMP}

If the selected controller has Ready drives available, creates an array.  (Information Mode only.)

# When you select a controller or array icon from the Main Tree and arrays are defined for the controller, deletes all of the arrays.  (Information Mode only.)

Attention:  After you delete the arrays from the controller configuration, you will not have access to any data stored on the logical drives attached to the selected ServeRAID adapter or controller.

# When you select a specific array icon from the Main Tree, deletes the array.  (Information Mode only.)

# If the logical drive is the last physically striped logical drive in the array, deletes the selected logical drive.  This option is only available if the logical drive is the last in the array.

# If set to the write-through mode, the completion status of a write command is sent after the data is written to the hard disk drive.  Under certain workloads, you can improve performance by changing this setting to the write-back mode, where the completion status of a write command is sent after the data is copied to cache memory, but before the data is actually written to the storage device.

If you have the optional battery-backup cache device installed, the default setting is write-back mode.  Otherwise, the default setting is write-through mode.  (Information Mode only.)

Attention:  1.  It is possible to lose data if a power outage occurs while using the write-back mode without a battery-backup cache device.

2.  If you do not have a battery-backup cache installed and enabled and you set this feature to the write-back mode, wait at least 10 seconds after your last operation before you turn off your system.  Failure to follow this practice can result in lost data.

# When the ServeRAID adapter or controller performs a Rebuild operation on an array, it reconstructs the data that was stored in RAID level-1 and RAID level-5 logical drives.  However, the ServeRAID adapter or controller can not reconstruct the data that was stored in any RAID level-0 logical drives in that array.  The data in the RAID level-0 logical drives is blocked when the ServeRAID adapter or controller detects that the array is valid, but the data might be corrupt.

After the Rebuild operation completes, you can unblock the RAID level-0 logical drives, and access them once again.  But remember, the logical drive might contain corrupt data.  You must either re-create, install, or restore the data from the most recent backup disk or tape.  (Information Mode only.)

# If the selected physical drive is in the Ready state and less than sixteen (16) hot-spare drives are defined, creates a hot-spare drive.  (Information Mode only.)

The system automatically uses the hot-spare drive when a similar drive fails.  The drive capacity of the hot-spare drive must be equal to or greater than the capacity of the drive being replaced.

# If the selected drive is in the Hot-Spare state, removes the hot-spare drive from the configuration by setting the drive to Empty.  If the drive is not actually removed, this action attempts to set the drive to Ready.  (Information Mode only.)

# When the selected drive is in the Defunct state, removes a defunct drive from the configuration by setting the drive to Empty.  (Information Mode only.)

A drive is in the defunct state if:

- A defunct driveDefunct_Drive_glossary has been rebuilt to another drive, such as a hot-spare drive.

- A hot-spare or standby hot-spare drive fails to respond to commands from the ServeRAID controller.

# When the selected drive is in the Defunct state, select this choice when you want to physically remove a defunct drive from the system and install a good drive in its place.  This action subsequently tries to set the new drive to be a hot-spare drive.  (Information Mode only.)

A drive is in the defunct state if:

- A defunct driveDefunct_Drive_glossary has been rebuilt to another drive, such as a hot-spare drive.

- A hot-spare or standby hot-spare drive fails to respond to commands from the ServeRAID controller.

# If the selected drive is in the Defunct Disk Drive state and the drive is part of a critical logical drive, automatically rebuilds a defunct drive when you replace it.

The ServeRAID controller rebuilds a defunct drive automatically when all of the following conditions exist:

- The hot-swap rebuild parameter is set to Enable (default).

- The defunct drive is part of a RAID level-1 or RAID level-5 logical drive.

- The capacity of the replacement drive is equal to or greater than the capacity of the defunct drive.

- No rebuild, synchronization, or RAID level change operation is in progress on this controller.

When these requirements are met, the ServeRAID controller automatically initiates a rebuild of the drive without user intervention.

# If the selected drive is in Defunct Disk Drive state and the drive is not part of a critical logical drive, sets the drive state to Online.  (Information Mode only.)

Attention:  You might lose data if by setting a Defunct physical drive that is part of an array to Online.

# For the selected SCSI Channel, changes the transfer speed from the default value of Optimal.  Choices available are Ultra2 SCSI, UltraSCSI, Fast SCSI 2, and SCSI 2.  (Information Mode only.)

# Click on the Apply button when you are satisfied with the configuration.

# Click on the Modify icon button to return to the Custom Configuration Physical Drive Screen.

# Click on the Modify icon button to return to the Custom Configuration Logical Drive Screen.

# Click on the OK button to accept any changes you have made.

# Indicates the progress of logical drive rebuild, logical drive migration, or synchronization.

# Program and copyright information for the IBM ServeRAID Configuration Program.

# {bmct tbhelp.BMP}

View task-oriented help for the current screen.

# If the system workload is random or the system I/O requests are smaller than the stripe-unit size, reading ahead to the end of the stripe might degrade performance.  When disabled, the controller transfers data from a disk to its local cache in increments equal to the system I/O request size, without reading ahead to the end of the stripe.

# Consider the following information for setting rebuild priority:

- When the rebuild priority is set to High, the rebuild I/O request gets highest priority in the execution order.

- When the rebuild request is set from High to Medium in a heavily loaded system, it can increase the disk rebuild time, but provide better system performance.

- When the rebuild request is set from High or Medium to Low in a moderate to heavily loaded system, it can increase the disk rebuild time, but provide better system performance.

# Select this choice to change the transfer speed from its default value of Optimal.  When the transfer speed is set to Optimal, the ServeRAID controller determines the best transfer speed, based on the types of SCSI drives and storage enclosures in use.  Choices available are Ultra2 SCSI, UltraSCSI, Fast SCSI 2, and SCSI 2.  (Information Mode only.)

# Sets the size of the chunk of data the controller reads from each drive at a time.  When you select 8 K (default) or 16 K, the maximum number of physical drives supported in an array is 16.  When you select 32 K or 64 K, the maximum number of physical drives supported in an array is 8.  (Information Mode only.)

To maximize overall performance, choose a size that is close to the size of the system I/O request.

Attention:   After you configure an array and store data on the logical drives, you cannot change the stripe-unit size without destroying data in the logical drives.

# If set to the write-through mode, the completion status of a write command is sent after the data is written to the hard disk drive.  Under certain workloads, you can improve performance by changing this setting to the write-back mode, where the completion status of a write command is sent after the data is copied to cache memory, but before the data is actually written to the storage device.

If you have the optional battery-backup cache device installed, the default setting is write-back mode.  Otherwise, the default setting is write-through mode.  (Information Mode only.)

Attention:  1.  It is possible to lose data if a power outage occurs while using the write-back mode without a battery-backup cache device.

2.  If you do not have a battery-backup cache installed and enabled and you set this feature to the write-back mode, wait at least 10 seconds after your last operation before you turn off your system.  Failure to follow this practice can result in lost data.

# Copies the configuration information stored on the hard disk drives to the ServeRAID controller.  Use this function to import previously configured drives from another system.

# If the selected physical drive is in the Ready state and less than sixteen (16) hot-spare drives are defined, creates a hot-spare drive.

The system automatically uses the hot-spare drive when a similar drive fails.  The drive capacity of the hot-spare drive must be equal to or greater than the capacity of the drive being replaced.

# When you select a controller or array icon from the Main Tree and arrays are defined for the controller, deletes all of the arrays.  (Information Mode only.)

Attention:  After you delete the arrays from the controller configuration, you will not have access to any data stored on the logical drives attached to the selected ServeRAID adapter or controller.

# When you select a specific array icon from the Main Tree, deletes the array.

Attention:  Data will be destroyed.

# If you select Yes, you will delete the selected logical drive.

Attention:  The data on the selected logical drive will be destroyed.

# Changes the ServeRAID adapter or controller settings (such as the stripe-unit size and rebuild rate) from their current or customized values to their factory default settings.

If there are no logical drives defined, the ServeRAID Configuration Program automatically restores all parameters in the configuration to the factory defaults.

If there are logical drives defined, click OK to delete all logical drives and restore all parameters in the configuration to the factory defaults. The ServeRAID Configuration Program will destroy all data on these logical drives.

# If the selected drive is in the Defunct Disk Drive state and the drive is part of a critical logical drive, automatically rebuilds a defunct drive when you replace it.

The ServeRAID controller rebuilds a defunct drive automatically when all of the following conditions exist:

- The hot-swap rebuild parameter is set to Enable.

- The defunct drive is part of a RAID level-1 or RAID level-5 logical drive.

- The capacity of the replacement drive is equal to or greater than the capacity of the defunct drive.

- No rebuild, synchronization, or RAID level change operation is in progress.

When these requirements are met, the ServeRAID controller automatically initiates a rebuild of the drive without user intervention.

# If the selected drive is in the Defunct state, select this choice when you want to physically remove a defunct drive from the system.

A drive is in the defunct state if:

- A defunct driveDefunct_Drive_glossary has been rebuilt to another drive, such as a hot-spare drive.

- A hot-spare or standby hot-spare drive fails to respond to commands from the ServeRAID controller.

# If the selected drive is in the Hot-Spare state, removes the hot-spare drive from the configuration by setting the drive to Empty.  If the drive is not actually removed, this action attempts to set the drive to Ready.  (Information Mode only.)

The system automatically uses the hot-spare drive when a similar drive fails.  The drive capacity of the hot-spare drive must be equal to or greater than the capacity of the drive being replaced

# If the selected drive is in the Defunct state, select this choice when you want to physically remove a defunct drive from the system and install a good drive in its place.  This action subsequently tries to set the new drive to be a hot-spare drive.

A drive is in the defunct state if:

- A defunct driveDefunct_Drive_glossary has been rebuilt to another drive, such as a hot-spare drive.

- A hot-spare or standby hot-spare drive fails to respond to commands from the ServeRAID controller.

# Reports the progress of the scan, the number of found Ready drives, and the number of removed Ready drives.

# If the selected drive is in Defunct Disk Drive state and the drive is not part of a critical logical drive, sets the drive state to Online.  (Information Mode only.)

Attention:  You might lose data if by setting a Defunct physical drive that is part of an array to Online.

# When the ServeRAID adapter or controller performs a Rebuild operation on an array, it reconstructs the data that was stored in RAID level-1 and RAID level-5 logical drives. However, the ServeRAID adapter or controller can not reconstruct the data that was stored in any RAID level-0 logical drives in that array.  The data in the RAID level-0 logical drives is blocked when the ServeRAID adapter or controller detects that the array is valid, but the data might be corrupt.

After the Rebuild operation completes, you can unblock the RAID level-0 logical drives, and access them once again.  But remember, the logical drive might contain corrupt data.  You must either re-create, install, or restore the data from the most recent backup disk or tape.

Attention:   A logical drive is blocked because it contains corrupt data.

# Starts the scan for new or removed Ready drives.

# Accepts the results of a scan for new or removed Ready drives and dismisses the dialog box.

# The Event Log displays information, attention, and error events as well as feedback from the polling process.  Every event has a date and time stamp and an icon.  The icons denote the severity of the event:

{bmc s_info.BMP}   Information

{bmct s_alert.BMP}   Alert

{bmct s_error.BMP}   Error

# Lets you work with a logical drive configuration on a selected array.  The note tab number is the logical drive number.  The color on the note tab matches the pie chart slice representing this logical drive on the array.

# If there is free space, adds another logical drive to the selected array.  When you click on this note tab, a logical drive is added.  This new logical drive is assigned the highest RAID level possible and uses all of the available free space.

# Lets you select a RAID level for the logical drive.

# Lets you specify the amount of space (in MBMB) for a logical drive.

# Deletes the specified logical drive on the specified array.  You cannot delete the last logical drive in the array.

# Lets you specify the space for a logical drive in the specified array by percentage.  Use the slider to adjust the size of the logical drive.

# System Information Main Tree for your server.

# System Information Main Panel for your server.

# Select a controller for configuration from this Main Tree.

# Select either the Express or Custom Configuration mode to configure the selected controller.

# Create arrays by dragging physical drives from the left-hand tree to an icon in the right-hand tree labeled either Add New Array or New Array.

Create hot-spare drives by dragging physical drives from the left-hand tree to an icon in the right-hand tree labeled Hot-Spare Drives.

# Select and change new items if they are not in the configuration yet.

# Select an array for which you want to create logical drives.

# Create the logical drives for the selected array.

# Expands and collapses so you can view everything on the ServeRAID controller.  You cannot select anything, only view.

# Presents a summary of the configuration for the ServeRAID controller.

# Click on the Apply button to create the new logical drive.

# Specify these configuration names to identify each node of a cluster.

For example, one node is named Server-A and its partner is named Server-B:

- On Server-A in the Controller Host ID entry field, you should specify Server-A.

- On Server-A in the Cluster Partner Host ID entry field, you should specify Server-B.

- On Server-B in the Controller Host ID entry field, you should specify Server-B.

- On Server-B in the Cluster Partner Host ID entry field, you should specify Server-A.

# Specify the SCSI IDs used for each shared channel on the controller.  Each shared channel ID must be unique on each cluster node.

# Specify the Merge-Group IDs.  They must be unique on each server, and must differ from each node in the cluster.

For example:

- A merge-group ID of 7 is assigned to Logical Drive 1 on Server-A.

- A merge-group ID of 8 is assigned to Logical Drive 1 on Server-B.

The number you choose to assign as the merge ID also depends on whether the drive is shared or not:

- If it is shared, the number is in the range of 1-8.

- If it is non-shared, the number is in the range of 201-215.

# # $ K @ Stripe-Unit Size and Interleave Depth

With RAID technology, data is striped across an array of hard disk drives.  This data-distribution scheme complements the way the operating system requests data.

The granularity at which data from one file is stored on one drive of the array before subsequent data is stored on the next drive of the array is called the interleave depth.

You can control the interleave depth and maximize the performance of your ServeRAID controller by setting a stripe-unit size that is close to the size of the system I/O requests.  You can set the stripe-unit size to 8 KB, 16 KB, 32 KB, or 64 KB.  For example, performance in transaction-based environments, which typically involve large blocks of data, might benefit when using a setting of 32 KB or 64 KB. However, performance in file and print environments, which typically involve multiple small blocks of data, might perform best when set to a stripe-unit size of 8 KB or 16 KB.

The collection, in logical order of these stripe units, from the first drive of the array to the last drive of the array, is called a stripe.

# # $ K A @ RAID Level-0

RAID level-0 stripesInterleave_Depth_and_Stripe_Unit_Size>gloss the data across all the drives in the array. This offers substantial speed enhancement, but provides for no data redundancy. RAID level-0 provides the largest capacity of the RAID levels offered, because no room is taken up for redundant data or data-parityParity storage.

The RAID level-0 requires a minimum of one drive and, depending upon the stripe-unit sizeStripe_Unit_Size>gloss, supports a maximum of eight or 16 drives.

# RAID level-0 example

| You have two physical drives. |  |
| --- | --- |
| Create an array with the physical drives. |  |
| Then, create a logical drive within the array. |  |
| The firmware stripes data across the drives, creating blocks. Notice that the data is striped across all the drives in the array, but no redundant data is stored. |  |

A hard disk failure within the array results in loss of data in the logical drive assigned RAID level-0, but only in that logical drive. If you have logical drives assigned RAID level-1RAID_Level_1>main, 1ERAID_Enhanced_Level_1>main, 5RAID_Level_5>main, or 5ERAID_Level_5E>main in the same array, they will not lose data.

Note: You can only assign RAID level-0 to logical drives in an array containing only one hard disk drive.

When you replace a failed drive, the ServeRAID controller can rebuild all the RAID level-1, 1E, 5, and 5E logical drives automatically onto the replacement hard disk drive. However, any data stored in a failed RAID level-0 logical drive is lost.

Though the risk of data loss is present, you might want to assign RAID level-0 to one of the logical drives to take advantage of the speed offered with this RAID level. You could use this logical drive to enter data that you back up each day and for which safety is not of primary importance, that is, data that you can re-create easily. You also might want to use a RAID level-0 logical drive when the work you are doing requires maximum capacity.

See Also

RAID level-1RAID_Level_1>main

RAID level-1ERAID_Enhanced_Level_1>main

RAID level-5RAID_Level_5>main

RAID level-5ERAID_Level_5E>main

Selecting a RAID levelSelecting_a_RAID_Level>main

# # $ K A @ RAID Level-1

RAID level-1 provides 100% data redundancy and requires only two hard disk drives. With RAID level-1, the first half of a stripeInterleave_Depth_and_Stripe_Unit_Size>gloss is the original data; and the second half of a stripe is a mirror (copy) of the data, but written to the other drive in the RAID level-1 array.

Because the data is mirrored, the capacity of the logical drive when assigned RAID level-1 is 50% of the array capacity of the total number of hard disk drives in the RAID level-1 array.

# RAID level-1 example

| You have two physical drives. |  |
| --- | --- |
| Create an array with the physical drives. |  |
| Then, create a logical drive within the array. |  |
| The firmware stripes data across the drives, creating blocks. Notice that the data on the right-hand drive is a mirror copy of the data on the left-hand drive. |  |

If one of the hard disk drives fails in the above example, the ServeRAID controller switches read and write requests to the remaining functional drive in the RAID level-1 array.

See Also

RAID level-0RAID_Level_0>main

RAID level-1ERAID_Enhanced_Level_1>main

RAID level-5RAID_Level_5>main

RAID level-5ERAID_Level_5E>main

Selecting a RAID levelSelecting_a_RAID_Level>main

# # $ K A @ RAID Level-1 Enhanced

If you have three or more hard disk drives in the array, you can no longer select RAID level-1, but you can select RAID level-1 Enhanced (also called RAID 1E).

The RAID level-1 Enhanced combines mirroring with data striping. This RAID level stripesInterleave_Depth_and_Stripe_Unit_Size>gloss data and copies of the data across all the drives in the array. As with the standard RAID level-1, the data is mirrored, and the capacity of the logical drive is 50% of the array capacity of the grouping of hard disk drives in the array.

The Enhanced RAID level-1 requires a minimum of three drives and, depending upon the stripe-unit sizeStripe_Unit_Size>gloss, supports a maximum of eight or 16 drives.

# RAID level-1 Enhanced example

| You have three physical drives. |  |
| --- | --- |
| Create an array with the physical drives. |  |
| Then, create a logical drive within the array. |  |
| The firmware stripes data across the drives, creating blocks. Notice that the stripe labeled is the data stripe and the stripe labeled is the mirror (that is, the copy) of the preceding data stripe. Also notice that each block on the mirror stripe is shifted one drive. |  |

If one of the hard disk drives fails in the above example, the ServeRAID controller switches read and write requests to the remaining functional drives in the RAID level-1 Enhanced array.

See Also

RAID level-0RAID_Level_0>main

RAID level-1RAID_Level_1>main

RAID level-5RAID_Level_5>main

RAID level-5ERAID_Level_5E>main

Selecting a RAID levelSelecting_a_RAID_Level>main

# # $ K A @ RAID Level-5

RAID level-5 requires a minimum of three hard disk drives. This RAID level stripesInterleave_Depth_and_Stripe_Unit_Size>gloss data and parityParity>gloss across all drives in the array. When an array is assigned RAID level-5, the capacity of the array is reduced by one drive (for data-parity storage).

RAID level-5 is generally the most desirable choice, because it offers both data protection and increased throughput. RAID level-5 gives you higher capacity than RAID level-1, but RAID level-1 offers better performance.

The RAID level-5 requires a minimum of three drives and, depending upon the stripe-unit sizeStripe_Unit_Size>gloss, supports a maximum of eight or 16 drives.

RAID level-5 example

| You have four physical drives. |  |
| --- | --- |
| Create an array with three physical drives, leaving the fourth as a hot-spare. |  |
| Then, create a logical drive within the array. |  |
| The firmware stripes data across the drives, creating blocks. Notice that the storage of the data parity (denoted by the ) also is striped, and it shifts from drive to drive. |  |
| A parity block () contains the data from the other blocks in the same stripe. |  |
| If a hard disk drive fails in the array, the ServeRAID controller switches read and write requests to the remaining functional drive in the RAID level-5 array. |  |

See Also

RAID level-0RAID_Level_0>main

RAID level-1RAID_Level_1>main

RAID level-1ERAID_Enhanced_Level_1>main

RAID level-5ERAID_Level_5E>main

Selecting a RAID levelSelecting_a_RAID_Level>main

# # $ + K A @ RAID Level-5 Enhanced

RAID level-5 Enhanced (also called RAID 5E) requires a minimum of four hard disk drives. RAID level-5E is also firmware-specific. You can think of RAID level-5E as "RAID level-5RAID_Level_5>main with a built-in spare drive."

Reading from and writing to four disk drives is more efficient than three disk drives and therefore improves performance. Additionally, the spare drive is actually part of the RAID level-5E (see the example below). With such a configuration, you cannot share the spare drive with other arrays. If you want a spare drive for any other array, you must have another spare drive for those arrays.

Like RAID level-5, this RAID level stripesInterleave_Depth_and_Stripe_Unit_Size>gloss data and parityParity>gloss across all drives in the array. When an array is assigned RAID level-5E, the capacity of the logical drive is reduced by the capacity of two physical drives in the array (that is, one for parity and one for the spare).

RAID level-5E is a very desirable choice, because it offers both data protection and increased throughput in addition to the built-in spare drive.

Note: For RAID level-5E, you can only have one logical drive in an array. When using RAID level-5E, you can have a maximum of only seven logical drives.

The Enhanced RAID level-5 requires a minimum of four drives and, depending upon the stripe-unit sizeStripe_Unit_Size>gloss, supports a maximum of eight or 16 drives.

# RAID level-5 Enhanced example

| You have four physical drives. |  |
| --- | --- |
| Create an array with all four physical drives. |  |
| Then, create a logical drive (labeled as 1) within the array. Notice that the distributed spare drive is the free space (labeled as 2) below the logical drive. |  |
| The firmware stripes data across the drives, creating blocks in the logical drive. The storage of the data parity (denoted by the) is striped, and it shifts from drive to drive as it does in RAID level-5. |  |
| Notice that the spare drive in the free space is not striped. |  |
| If a hard disk drive fails in the array, the data from the failed drive is compressed into the spare drive. The logical drive remains RAID level-5 Enhanced. When you replace the failed drive, the data for the logical drive decompresses and returns to the striping scheme as seen in the previous picture. |  |

Note: Express Configuration does not default to RAID level-5E. If the user has four disk drives, Express Configuration defaults to RAID level-5 plus a hot-spare.

See Also

RAID level-0RAID_Level_0>main

RAID level-1RAID_Level_1>main

RAID level-1ERAID_Enhanced_Level_1>main

RAID level-5RAID_Level_5>main

Selecting a RAID levelSelecting_a_RAID_Level>main

# # $ K @ Hard Disk Drives Capacities

Hard disk drive capacities influence the way you create arrays.  Drives in the array can be of different capacities (1 GBGB or 2 GB, for example), but the ServeRAID controller treats them as if they all have the capacity of the smallest disk drive.

For example, if you group three 1 GB drives and one 2 GB drive into an array, the total capacity of the array is 1 GB times 4, or 4 GB, not the 5 GB physically available. The remaining space on the 2 GB drive is unusable capacityUnusable_Capacity.

Conversely, if you add a smaller drive to an array of larger drives, such as a 1 GB drive to a group containing three 2 GB drives, the total capacity of that array is 4 GB, not the 7 GB physically available.  Therefore, the optimal way to create arrays is to use hard disk drives that have the same capacity.

A hot-spare drive also must be at least as large as the smallest drive in the array.

# $ K @ GB

When referring to hard-disk-drive capacity, GB means 1 000 000 000 bytes, or 1000 MB.  Total user-accessible capacity may vary depending on operating environment.

# # $ K @ MB

When referring to hard-disk-drive capacity, MB means 1 000 000 bytes.  Total user-accessible capacity may vary depending on operating environment.

# $ K @ Logical Drives

When you create an array, you group hard disk drives into one storage area.  You can define this storage area as a single logical drive, or you can subdivide it into several logical drives.  Each logical drive appears to the operating system as a single physical hard disk drive.

The ServeRAID controller supports up to eight logical drives.  If you have only one array, you can define it as a single logical drive, or you can divide it into several logical drives.  Typically, the first logical drive that you define will be your startup (boot) drive.  If you have two or more arrays, each array can be one logical drive, or you can divide each array into multiple logical drives, as long as the total number of logical drives for all of the arrays does not exceed eight.

# # $ Defunct physical drive

{bmlt s_pderror.BMP}

The physical hard disk drive is defunct.  A drive in the Online, Hot-Spare, or Rebuilding state has become defunct.  It does not respond to commands, which means that the ServeRAID controller cannot communicate properly with the drive.

A hard disk drive in the Defunct state does not necessarily mean that you need to replace the drive.  Before you replace the drive, ensure that:

- All cables are connected correctly to the backplane and to the hard disk drive.  Also, check to ensure that all cables inside the server are connected correctly.

- The hot-swap drive tray is seated properly in the drive bay.

# # $ Hot-Spare physical drive

{bmlt s_pdhsp.BMP}

A hot-spare drive is a hard disk drive that is defined for automatic use when a similar drive fails.  The drive capacity of the hot-spare drive must be equal to or greater than the capacity of the drive being replaced.

Notes:

- If you want to have a hot-spare drive or a standby hot-spare drive and also assign RAID level-5 to one of your logical drives, you must have at least four hard disk drives.

- In the event of a drive failure in an array that contains only logical drives assigned RAID level-0, the ServeRAID controller will not activate a hot-spare drive.

# # $ K @ Ready physical drive

{bmct s_pdrdy.BMP}

The ServeRAID controller recognizes a Ready drive as being available for definition.

# # $ K @ Online physical drive

{bmlt s_pdonl.BMP}

The drive is Online.  It is functioning properly and is part of an array.

# # $ K @ Standby physical drive

{bmct s_pdrdy.BMP}   {bmct s_cdrom.BMP}   {bmct s_tape.BMP}   {bmct s_proc.BMP}

A Standby drive is a hard disk drive, CD-ROM, tape, removable media, or enclosure that the ServeRAID controller has spun down.

# # $ K @ Rebuilding physical drive

{bmlt s_pdalert.BMP}

The drive is being rebuilt.

# # $ K @ Critical Migrating Logical Drive

A logical drive undergoing a logical-drive migration is in the critical state.

# # $ K @ Critical System Logical Drive

The ServeRAID controller uses this reserved state during a logical-drive migration and the logical drive is in the critical state.

# # $ K @ Critical Logical Drive

A RAID level-1 or 5 logical drive that contains a defunct physical drive is in the critical state.  A critical logical drive is accessible, despite a physical drive failure.

If the state of the logical drive is critical, you must replace and rebuild the defunct drive.

# # $ K @ Migrating Logical Drive

A logical drive undergoing a logical-drive migration is in a non-critical state.

# $ K @ Offline Logical Drive

The logical drive is offline and not accessible.  This state occurs if one or more physical drives in a RAID level-0 logical drive are defunct.  This state also occurs when two or more physical drives in a RAID level-1 or RAID level-5 logical drive are defunct.

# # $ K @ Okay Logical Drive

The drive is okay.  It is in a good, functional state.

# # $ K @ System Logical Drive

The ServeRAID controller uses this reserved state during logical-drive migration.

# # $ K @ Stripe-Unit Size

The stripe-unit size is the amount of data written on a given disk before writing on the next disk.  To maximize the overall performance, choose a size that is close to the size of the system I/O request.

Note:  The size of the system I/O request depends on your operating system.  If you are not sure of the size, use the default setting.

Attention:  After you configure an array and store data on the logical drives, you cannot change the stripe-unit size without destroying data in the logical drives.

You can set the stripe-unit size to 8 K, 16 K, 32 K, or 64 K.  The default setting is 8 K data bytes.

- When the stripe-unit size is 8 K or 16 K, the maximum number of physical drives in an array is 16.

- When the stripe-unit size is 32 K or 64 K, the maximum number of physical drives in an array is 8.

# # $ K @ Adaptive Read-Ahead Cache Mode

The ServeRAID controller continually reevaluates whether to transfer data from disk to its local cache in increments equal to the stripe-unit size or in increments equal to the system I/O request size.

The ServeRAID-3H and 3L controllers, as well as the new ServeRAID-3HB, use new firmware to implement Adaptive Read-Ahead Cache mode. You can downloadFinding_Information_on_the_World_Wide_Web>main and install the new firmware for the ServeRAID-3H and 3L controllers. The ServeRAID-3HB option comes with the new firmware.

If you have the controller and firmware for Adaptive Read-Ahead mode installed, the ServeRAID Configuration Program reports "Adaptive” on the controller's information panel. Otherwise, the information panel states either Enabled or Disabled.

Note: With this new firmware, the non-Adaptive Read-Ahead modes are no longer available in the ServeRAID Configuration Program. You can override Adaptive Read-Ahead, but not through the ServeRAID Configuration Program. Instead, use the IPSSEND utility. This utility comes with the device option and is on diskette.

# # $ K @ Understanding the Tree View Icons

{bmct s_server1.BMP}	Server

{bmlt s_adapt1.BMP}	Controller

{bmct s_arrays1.BMP}	Arrays (plural)

{bmct s_logdrs1.BMP}	Logical Drives

{bmct s_hsps1.BMP}	Hot-Spare Drives

{bmct s_phdrs1.BMP}	Physical Drives

{bmct s_scsi1.BMP}	SCSI Channel

{bmct s_array1.BMP}	Array

{bmct s_logdr1.BMP}	Logical Drive

{bmct s_pdrdy.BMP}	Ready Drive

{bmct s_pdonl.BMP}	Online Drive

{bmct s_pdhsp.BMP}	Hot-Spare Drive

{bmct s_cdrom.BMP}	CD-ROM Drive

{bmct s_tape.BMP}	Tape Drive

{bmct s_proc.BMP}	Enclosure

{bmct s_pdrdy.BMP}	Removable Media Drive

The icons above can have alert and error icons superimposed on top to indicate alert and error conditions.

{bmct s_alert.BMP}	Alert (for example, {bmct s_adapt2.BMP} this icon represents an alert on the controller.)

{bmct s_error.BMP}	Error (for example, {bmct s_adapt3.BMP} this icon represents an error on the controller.)

# # $ K @ Event Log Icons

The Event Viewer displays an icon with every event to denote the severity of the event:

{bmc s_info.BMP}   Information

{bmct s_alert.BMP}   Alert

{bmct s_error.BMP}   Error

# # $ K A @ Server System Information Field Definitions

{bmlt s_server1.BMP}

- System Name reports the system name of the server.

- Number of ServeRAID Controllers reports the number of ServeRAID controllers in the server.

# # $ K A @ Controller System Information Field Definitions

{bmlt s_adapt1.BMP}

- Controller Type reports whether the controller is ServeRAID, ServeRAID II, ServeRAID 3H, ServeRAID 3L, or ServeRAID on the system board.  The type is only shown if your BIOS is version 3.0 or later.

- ServeRAID Number is the ServeRAID Controller number.  The number is only shown if the BIOS is earlier than version 3.0.

- BIOS Version reports the version of the BIOS on the controller.  This information is only displayed if the BIOS version is 3.0 or later.

- Firmware Version reports the version of the ServeRAID firmware.

- Physical Slot shows the actual physical slot location for the ServeRAID controller; for example, PCI slot 4.  The slot is only shown if your BIOS is version 3.0 or later.

- Battery Backup Write Cache reports whether this optional device is Installed, Not Installed, Defective, or needs a battery replacement.

- Read-Ahead Cache ModeRead_Ahead_Cache_Mode reports whether this feature is enabled, disabled, or adaptiveAdaptive_Read_Ahead_Mode.

- Stripe-Unit Size reports the size setting of 8 K, 16 K, 32 K, or 64 K.

- Rebuild Rate reports the rate setting of High, Medium, or Low.

- Hot-Swap RebuildHot_Swap_Rebuild reports this feature as enabled.  The setting is only shown if the ServeRAID firmware supports Hot-Swap Rebuild.

- Data ScrubbingData_Scrubbing reports this feature as enabled.  The setting is only shown if the ServeRAID firmware supports Data Scrubbing.

- Auto-SynchronizationAuto_Synchronization reports this feature as enabled.  The setting is only shown if the ServeRAID firmware supports Auto-Synchronization.

- ClusteringCluster reports Enabled or Disabled based on the Cluster Partner ID and Merge Group IDs configuration, if the ServeRAID firmware supports clustering.

- Host Configuration ID reports the ID if the controller is part of a cluster.

- Cluster Partner ID reports the ID if the controller is part of a cluster.

- Unattended ModeUnattended_Mode reports whether this feature is enabled or disabled.

- Number of Arrays reports the number of defined arrays.

- Number of Logical Drives reports the number of defined logical drives.

- Number of Hot-Spare Drives reports the number of defined hot-spare drives.

# # $ Arrays Information

{bmlt s_arrays1.BMP}

Each defined array appears with the following information:

- Array Letter

- Size in MBMB

- Free Space in MB

- Number of Logical Drives

# # $ K A Logical Drives Information Field Definitions

{bmlt s_logdrs1.BMP}

Each defined logical drive appears with the following information:

- Drive Number

- Array Letter

- Size in MBMB

- RAID Level

# # $ Physical Drives Information

{bmlt s_phdrs1.BMP}

For each channel that has at least one drive, reports the number of drives on the channel.

# # $ K A @ Array Query Information Field Definitions

{bmlt s_array1.BMP}

- Array Letter is the letter of the selected array.

- Array Size in MB is the total size, in MBMB.

- Free Space in MB is the amount of free space available, in MB.

- Number of Logical Drives is the number of logical drives in the selected array.

- Number of Physical Drives is the number of physical drives in the selected array.

# # $ K Hot-Spare Drives Information

{bmlt s_hsps1.BMP}

Reports the total number of hot-spare drives.

# # $ K A @ Logical Drives in Array Query Information Field Definitions

{bmlt s_logdrs1.BMP}

Each defined logical drive within the specified array appears with the following information:

- Logical Drive Number

- Array Letter

- RAID Level

- Size in MBMB

# # $ Physical Drives in Array Query Information

{bmlt s_phdrs1.BMP}

For each channel that has at least one drive in the selected array, reports the number of drives in the selected array that are on the channel.

# # $ K A Logical Drive Query Information Field Definitions

{bmlt s_logdr1.BMP}

- Logical Drive Number reports the number of the selected logical drive as defined in the controller.

# Array Letter reports the letter of the array that contains the selected logical drive.

- Drive State reports whether the logical drive state is OkayOkay_Logical_Drive, CriticalCritical_Logical_Drive, Critical MigratingCritical_Migrating_Logical_Drive, Critical SystemCritical_System_Logical_Drive, Migrating,Migrating_Logical_Drive OfflineOffline_Logical_Drive, SystemSystem_Logical_Drive, or Blocked.Blocked_Access

- RAID Level reports the logical drive RAID level as 0RAID_Level_0>main, 1RAID_Level_1>main, or 5RAID_Level_5>main.  Enhanced RAID level-1RAID_Enhanced_Level_1>main is reported as 1.

- Data Space in MB reports the available space for data storage in this logical drive.

- Parity Space in MB reports the allocated space for parity storage in the array.

- Date Created reports the date the logical drive was created.

- Write-Cache ModeWrite_Cache_Mode reports whether the logical drive is set to write through or write back.

- Battery Backup Write Cache reports whether this option is enabled or disabled.  The Battery Backup Write Cache is enabled if a Battery Backup CacheBattery_Backup_Cache device is installed and the logical drive write-cache mode is set to write-back.  Otherwise, this option is disabled.

- Merge-Group Number displays if the controller is part of a cluster.

- Merge-Group State displays shared or non-shared if the controller is part of a cluster.

# # $ K A Physical Drive Information Field Definitions

{bmlt s_pdonl.BMP}

- SCSI ID reports the selected physical drive SCSI ID.

- Channel reports the channel to which the selected physical drive is connected.

- Drive Type reports whether the physical drive is a hard disk, CD-ROM, tape, removable-media, or an enclosureEnclosure.

- Size in MB reports the total size of the physical drive, in MBMB.  This information does not appear for CD-ROM, tape, or processor devices.

- Drive State reports the physical drive state as ReadyReady_Drive, OnlineOnline_Drive, DefunctDefunct_Drive_glossary, RebuildingRebuild_glossary, StandbyStandby_Drive, or Hot SpareHot_Spare_Drive_glossary.  This information only appears for hard disk drives.

- Array Letter if the drive is defined in an array, reports the letter (A through H) of the array that contains the selected physical drive.

- Vendor shows characters that represent the drive manufacturer.

- Product or Model Number shows characters that represent the product or model number for the drive.

- Serial Number shows the serial number of the drive.

- Drive Firmware Level shows the current level of firmware microcode in use for the drive.

- PFA ErrorPFA_Error indicates the status, Yes or No, for predictive-failure alerts.  Yes indicates that PFA errors were detected, and No indicates that PFA errors were not found.

# # $ K A Channel Information Field Definitions

{bmlt s_scsi1.BMP}

- Number of Drives reports the number of drives on the SCSI channel.

- SCSI Initiator ID reports the initiator ID of the SCSI channel.

- SCSI Transfer Speed reports the speed as Optimal, Ultra2 SCSI, UltraSCSI, Fast SCSI-2, SCSI-2, or SCSI-1.

# # $ K Clustering

A set of two or more computers that work as a group for greater performance, reliability, and prevention of failure.

# # $ K @ Battery Backup Cache

An optional device that stores the data currently being written into a battery.

# # $ K @ Hot-Swap Rebuild

If you replace a dead drive, this option enables an automatic rebuild onto the new drive.

# # $ K @ Auto-Synchronization

If enabled, the controller automatically synchronizes all of the RAID level-5 logical drives, in the background, when they are created.  RAID level-0 and RAID level-1 logical drives do not need to be synchronized when they are created.

# # $ K @ Data Scrubbing

When enabled, provides automatic, continuous synchronization during system use.

# # $ K @ PFA Error

Predictive Failure Analysis®Copyright_and_Trademarks_1 Copyright_and_Trademarks_1 is Copyright_and_Trademarks_1an event that announces the physical drive is about to fail.

# # $ K @ Blocked Logical Drive

During a rebuild operation, the ServeRAID controller sets the state of any RAID level-0 logical drives associated with a failed array to the Blocked state; then, it reconstructs the data that was stored in RAID level-1 and RAID level-5 logical drives.

After the Rebuild operation completes, you can unblock the RAID level-0 logical drives, and access them once again.  However, the logical drive might contain corrupt data.  You must either re-create, install, or restore the data from the most recent backup disk or tape to the RAID level-0 logical drive.

# # $ K @ Write-Cache Mode

Determines whether the controller writes data to the hard disk drive before or after sending the operating system a successful reply to the write command.  You can select from write backWrite_Back or write throughWrite_Through.

# # $ K @ Write Back

In write-back mode, the operating system sends data to the controller to write to a storage device.  Subsequently, the controller sends a reply to the write command before actually writing the data to the storage device.  Doing so increases performance, but also contains an element of risk.  For example, if there is a power failure the data currently in the controller’s cache is lost.  You can avoid this risk by installing an optional battery-backup cache device.  This device preserves the data in the controller’s cache in the event of a power failure.

Attention:  1.  It is possible to lose data if a power outage occurs while using the write-back mode without a battery-backup cache device.

2.  If you do not have a battery-backup cache installed and enabled and you set this feature to the write-back mode, wait at least 10 seconds after your last operation before you turn off your system.  Failure to follow this practice can result in lost data.

# # $ K @ Write Through

In write-through mode, the operating system sends data to the controller to write to a storage device.  Subsequently, the controller writes the data to the storage device; then, sends a reply to the write command.  This mode can decrease performance, but contains no risk of losing data.

# # $ K @ Could not determine the number of ServeRAID controllers.

Power down the server, reseat the controller, and restart the server.  If the error persists, call your IBM Service Representative.

# # $ K @ Could not apply the complete configuration to ServeRAID...

Power down the server, reseat the controller, and restart the server.  Initialize the configuration.  If the error persists, call your IBM Service Representative.

# # $ K @ Could not create the new logical drive on ServeRAID...

Power down the server, reseat the controller, and restart the server.  Initialize the configuration.  If the error persists, call your IBM Service Representative.

# # $ K @ Synchronize Drives

Synchronizing a logical drive verifies that the data redundancy for the logical drive is correct.   You must synchronize all RAID level-5 logical drives before storing data.

Synchronizes all new RAID level-5 logical drives when the following conditions are both true:

- The drive is attached to a ServeRAID adapter or controller that does not support the auto-synchronization feature

- You have not disabled the synchronize setting in the Options dialog on the View menu.

This is a lengthy operation.  Do not power-off the server until the synchronization is complete.

# # $ K @ Options

Use this dialog box to change the Initialization and Synchronization Mode, as follows:

Basic:  This is the default setting.  When this mode is Enabled, the configuration program automatically initializes each new logical drive and prevents access to the manual initialization and synchronization functions.  This choice also forces the synchronization of all new RAID level-5 logical drives attached to older ServeRAID adapter or controller models that do not support the auto-synchronization feature.

Advanced:  When Enabled, this choice lets you select to automatically initialize new logical drives and force the synchronization of all new RAID level-5 logical drives before exiting the ServeRAID Configuration Program.

Attention:  If you disable Automatic Initialization, the ServeRAID Configuration Program will not initialize each logical drive (by writing zeros to the first 1024 sectors of the logical drive) when it is created.  You must still initialize all newly defined logical drives before storing data.  If you disable the synchronization feature and the level of firmware being used by the ServeRAID adapter or controller does not support the auto-synchronization feature, you still must synchronize all RAID level-5 logical drives before storing data.

# Select this option to initialize (by writing zeros to the first 1024 sectors) the selected logical drive.

# Verifies that the data redundancy for the logical drive is correct.  You must synchronize all RAID level-5 logical drives before storing data.  (Information Mode only.)

# Click on the OK button to acknowledge the system message.

# # $ K A Finding Information on the World Wide Web

| IBM Home Page | http://www.ibm.com/pc/ |
| --- | --- |
| IBM Support Page From this page you can select: - IBM Server Support - Server Options (under family) - Downloadable Files - PCI RAID to download newer versions of the code. | http://www.ibm.com/pc/support/ |
| IBM Server Compatability Page Detailed information about server compatability issues. | http://www.ibm.com/pc/us/compat/ |
| IBM Clustering Detailed instructions for the clustering features are provided in the Netfinity®Copyright_and_Trademarks_1 High-Availability Cluster Solutions Installation and User's Guide which is under online publications. | http://www.ibm.com/pc/support/ - Select Server from the Select a Brand drop-down list box. - In the Select a Family list box, select Clustering. |

# # $ Enclosure

A SCSI management device that requires the use of an ID, such as an EXP15.

# # $ K Free Space

The space currently not assigned to a logical drive.

# # $ Configuration Mode

In the Configuration Mode, you can create or delete disk arrays, create or delete logical drives, and define your hot-spare drives.  When this mode is active, only a limited set of functions are available from the Menu and Tool Bars.

# # $ Information Mode

In the Information Mode, you can view the existing device and configuration information for your ServeRAID subsystem.  When this mode is active, you can use the functions available from the Menu and Tool Bars to customize settings for your ServeRAID controllers.

# Synchronizing a logical drive verifies that the data redundancy for the logical drive is correct.  You must synchronize all RAID level-5 logical drives before storing data.

Synchronizes the selected logical drive (from the Main Tree) when the following conditions are both true:

- The drive is attached to a ServeRAID adapter or controller that does not support the auto-synchronization feature

- You previously selected Options from the View pull-down menu and enabled Advanced mode.

This is a lengthy operation.  Do not power-off the server until the synchronization is complete.

# # $ + K A @ Performance Tuning Your System

When tuning your ServeRAID settings for optimal performance, consider the applications you intend to run on your server. ServeRAID settings are usually sensitive to the types of application running on the server, not its workload intensity or the number of users using the server.

When configuring your ServeRAID controller, your server's application environment helps determine the controller's stripe-unit size settings.

| EnvironmentApplication_Environment_Groups_Reference>gloss | Stripe-unit sizeStripe_Unit_Size>main |
| --- | --- |
| Groupware | 16 KB |
| Transaction processing database | 16 KB |
| Decision support database | 16 KB |
| Thin client environments | 8 KB |
| File server | 16 KB |
| Web server | 8 KB |
| Other | Default |

Your application environment can also influence the RAID level you select for your server. You must investigate your application's design, especially its I/O behavior, to make your decision.

See Also

Selecting a RAID levelSelecting_a_RAID_Level>main

Examples of application environment groupsApplication_Environment_Groups_Reference>main

Write-back modeWrite_Back>gloss

Write-through modeWrite_Through>gloss

Stripe-unit sizeInterleave_Depth_and_Stripe_Unit_Size>main

Adaptive read-ahead cache modeAdaptive_Read_Ahead_Mode>main

# # $ + K Application Environment Groups Reference

| Environment | Applications |
| --- | --- |
| Groupware | Lotus Notes®Copyright_and_Trademarks_1 MicrosoftÒ Exchange Other |
| Transaction processing | DB2Ò Informix Oracle SQLServer Sybase Other |
| Decision support or data warehousing | DB2 Informix Oracle SQLServer Sybase Other |
| Thin client environments | Citrix WinFrame or MetaFrame Microsoft Terminal Server Other |
| File server | Netware Microsoft Windows NTÒ Other |
| Web server | Apache Microsoft IIS Netscape Commerce Server Other |
| Other applications |  |

# # $ + K @ Critical State Logical Drive

A RAID level-1, 1E, 5, or 5E logical drive that contains a defunct physical drive is in the critical state. A critical logical drive is accessible, despite a physical drive failure.

If the state of the logical drive is critical, you must replace and rebuild the defunct drive.

# # $ + K High-Availability

A system designed and implemented with components that satisfy the system's functional requirements, but also has hardware and software component redundancy to eliminate any unexpected behavior.

# # $ + K Parity

The information stored on a RAID level-5 or level-5 Enhanced logical drive that holds the redundant data necessary to rebuild a failed drive.

# # $ + K A Total Disk Space

Hard disk drives in an array can be of different capacities (1 GB, or 2 GB, for example). The sum of the hard disk drives' capacity grouped in an array is the total disk space.

For example, if you group two 2 GB drives and one 3 GB drive into an array, the total disk capacity is the 7 GB physically available.

{bmct d3mg3a00.bmp}

Similarly, if you group three 2 GB drives and 1 GB drive into an array, the total disk space is the 7 GB physically available.

# # $ + K A Unusable Capacity

Hard disk drive capacities influence the way you create arrays. Drives in an array can be of different capacities (1 GB, or 2 GB, for example), but RAID controllers treat them as if they all have the capacity of the smallest disk drive.

For example, if you group two 2 GB drives and one 3 GB drive into an array, the usable capacityUsable_Capacity>gloss of the array is 2 GB times 3, or 6 GB, not the 7 GB physically available. The 7 GB is the total disk spaceTotal_Disk_Space>gloss. The remaining space on the 3 GB drive is unusable capacity. In the following diagram, the usable capacity is labeled as 1 and the unusable capacity is labeled as 2.

{bmct d3mg3a05.bmp}

Similarly, if you group three 2 GB drives and 1 GB drive into an array, the usable capacity of that array is 4 GB, not the 7 GB physically available. The 7 GB is the total disk space. The remaining space left on the three 2 GB drive is unusable capacity.

The optimal way to create arrays is to use hard disk drives that have the same capacity. Doing so avoids unusable capacity.

# # $ + K Usable Capacity

Hard disk drive capacities influence the way you create arrays. Drives in an array can be of different capacities (1 GB, or 2 GB, for example), but RAID controllers treat them as if they all have the capacity of the smallest disk drive.

For example, if you group two 2 GB drives and one 3 GB drive into an array, the usable capacity of the array is 2 GB times 3, or 6 GB, not the 7 GB physically available. The 7 GB is the total disk spaceTotal_Disk_Space>gloss. In the following diagram, the usable capacity is labeled as 1 and the unusable capacity is labeled as 2.

{bmct d3mg3a05.bmp}

Similarly, if you group three 2 GB drives and 1 GB drive into an array, the usable capacity of that array is 4 GB, not the 7 GB physically available. The 7 GB is the total disk space. The remaining space left on the three 2 GB drive is unusable capacity.

The optimal way to create arrays is to use hard disk drives that have the same capacity. Doing so avoids unusable capacityUnusable_Capacity>gloss.

# # $ + K Hot Spare

A hot-spare drive is a physical drive that is defined for automatic use when a similar (that is, with equal or smaller capacity) drive that is part of the array fails.

# # $ + K A Adaptive Read-Ahead Mode

The ServeRAID controller continually reevaluates whether to transfer data from disk to its local cache in increments equal to the stripe-unit size or in increments equal to the system I/O request size.

The ServeRAID-3H and 3L controllers, as well as the new ServeRAID-3HB, use new firmware to implement Adaptive Read-Ahead Cache mode. You can downloadFinding_Information_on_the_World_Wide_Web>main and install the new firmware for the ServeRAID-3H and 3L controllers. The ServeRAID-3HB option comes with the new firmware.

If you have the controller and firmware for Adaptive Read-Ahead mode installed, the ServeRAID Configuration Program reports "Adaptive” on the controller's information panel. Otherwise, the information panel states either Enabled or Disabled.

Note: With this new firmware, the non-Adaptive Read-Ahead modes are no longer available in the ServeRAID Configuration Program. You can override Adaptive Read-Ahead, but not through the ServeRAID Configuration Program. Instead, use the IPSSEND utility. This utility comes with the device option and is on diskette.

# # $ + K A Using Identify to Work with Your System

When you select this action for an icon in the Main Tree, the ServeRAID Manager makes the physical disk drive associated with the selected icon blink. That is, the physical disk drive light blinks to readily identify itself to you from within the server.

Depending on what you select in the Main Tree view, you can identify:

- A drive

- All drives

- A channel of drives

- A logical drive

- All logical drives

- An array

- All arrays.

You can only make one selection at a time. The hardware will continue blinking until you click OK on the message box.

Identify only works on controllers with a SAF-TE processor and the appropriate firmware. The processor uses a SCSI ID, therefore, it displays in the Main Tree as the following:

Enclosure

If a drive light or series of drives' lights do not blink, it is because all or part of the subsystem is not controlled by a SAF-TE processor.

For example, if SCSI Channel 1 has a SAF-TE processor, but SCSI Channel 3 does not, and you select to identify all physical drives, only SCSI Channel 1 will blink.

Identify is particularly useful for enclosure managers, such as the Netfinity®Copyright_and_Trademarks_1  EXP15. You can use Identify to clarify which drives in the enclosure you need to work with, before you pull them out of the enclosure.

If a SAF-TE processor is present and the controller supports drive identification, then:

| Right-mouse click on: |  | To identify... |
| --- | --- | --- |
|  | Controller | All the physical drives on the controller. |
|  | Arrays | All the arrays. |
|  | Logical Drives | All the logical drives. |
|  | Physical Drives | All physical drives. |
|  | Hot-Spare Drives | All the hot-spare drives. |
|  | Array ? | All the physical drives in the selected array. |
|  | Logical Drive ? | All the physical drives in the selected logical drive. |
|  | Logical Drives in Array ? | All the physical drives in the selected logical drives. |
|  | Physical Drive ? | The selected physical drive. |
|  | Physical Drives In Array ? | All the physical drives in the selected array. |
|  | Channel ? | All the physical drives on the selected channel. |

# # $ + K A Restore to Factory Defaults

Changes the ServeRAID adapter or controller settings (such as the stripe-unit size and rebuild rate) from their current or customized values to their factory default settings.

If there are no logical drives defined, the ServeRAID Configuration Program automatically restores all parameters in the configuration to the factory defaults.

If there are logical drives defined, click OK to delete all logical drives and restore all parameters in the configuration to the factory defaults. The ServeRAID Configuration Program will destroy all data on these logical drives.

# K Makes the hardware associated with the selected icon blink. That is, the hardware light blinks to readily identify itself to you from within the server.

# # $ + K Unattended Mode

Unattended mode is a setting that alters how the ServeRAID startup code (that is, BIOS) handles failures during a system startup. Examples of possible failures during the BIOS are the following:

- The BIOS cannot communicate with the controller

- A physical drive state has changed

- A logical drive state has changed.

You can set unattended mode either Disabled or Enabled. The default is Disabled.

When unattended mode is disabled and the BIOS detects a failure, the system remains at the recovery option screen and waits for the user to respond.

When unattended mode is enabled and the BIOS detects a failure, the system waits for the user to respond within 30 seconds to the recovery options screen. If no user responds, the BIOS automatically selects the first recovery option in the list and continues the system startup process. This mode is useful for remote systems where a user is not present for system startups.

You should carefully consider whether to enable unattended mode. Depending on how you use the system, unattended mode can be undesirable.

For example, consider a system that includes an enclosure of multiple hard disk drives such as the Netfinity®Copyright_and_Trademarks_1 EXP15. If the enclosure is turned off when you start the system, the BIOS reports a drive failure. If unattended mode is enabled, the BIOS accepts a recovery option that changes the configuration. When the system restarts with the enclosure powered on, the configuration is no longer correct and the system no longer works as designed.

Note: If you are configuring your system for clustering, you must enable unattended mode. If you use the ServeRAID Configuration Program to enable your system for clustering, the ServeRAID Configuration Program automatically enables unattended mode.

#Alters how the ServeRAID startup code (that is, BIOS) handles failures during a system startup. You can set unattended mode either Disabled or Enabled. The default is Disabled.

# Merges or unmerges logical drives associated with a failed controller within a cluster.

# # $ + K Merging and Unmerging Logical Drives

The ServeRAID controller supports two systems in a cluster. When a controller fails in a system that is part of a cluster, you must replace the controller.

For detailed information about replacing a controller, refer to the IBM Netfinity High-Availability Cluster Solutions Installation and User’s GuideFinding_Information_on_the_World_Wide_Web>main.

# Merging a Logical Drive

- Select the controller for the cluster from the Main Tree.

- Select Merge or Unmerge Logical Drive from the Actions menu.

- Select from the cascading menu whether to merge a logical drive that is one of the following:

- A logical drive on the cluster partner system and set to Shared.

- A logical drive on the selected controller's system (that is, your own system) and set to Shared.

- A logical drive on the selected controller's system (that is, your own system) and set to Non-shared.

- Select from the cascading menu the Merge Group number associated with the logical drive to merge.

- Click Yes to confirm merging the logical drive on the selected controller.

# Unmerging a Logical Drive

- Select the controller for the cluster from the Main Tree.

- Select Merge or Unmerge Logical Drive from the Actions menu.

- Select from the cascading menu whether to unmerge a logical drive that is one of the following:

- A logical drive on the selected controller's system (that is, your own system) and set to Shared.

- A logical drive on the selected controller's system (that is, your own system) and set to Non-shared.

- Select from the cascading menu the Merge Group number associated with the logical drive to unmerge.

- Click Yes to confirm unmerging the logical drive on the selected controller.

# # $ + K What's New in the ServeRAID Configuration Program

- RAID Level 5E (that is, Enhanced)RAID_Level_5E>main

- Identify drivesIDH_DLG_Identify_Drives>main

- Restore to factory defaultsRestore_to_Factory_Defaults>main

- Unattended mode for clustering supportUnattended_Mode>main

- Merging or unmerging logical drivesMerge_Unmerge_Logical_Drives>main

- Support for up to fourteen controllers

# # $ K Copyright

© IBM Corporation 1999. All rights reserved. U.S. Government Users Restricted Rights - Use, duplication or disclosure restricted by GSA ADP Schedule Contract with IBM Corp.

# Trademarks

The following terms are trademarks of International Business Machines Corporation in the United States, or other countries,or both:

| IBM | Predictive Failure Analysis |
| --- | --- |
| Netfinity | ServeRAID |

Microsoft, Windows, and Windows NT are registered trademarks of Microsoft Corporation.

Other company, product, and service names may be trademarks or service marks of others.

# # $ + K SAF-TE

A processor that manages small computer system interface (SCSI) accessed fault-tolerant enclosures (SAF-TE*Copyright_and_Trademarks_1). An example of such an enclosure is the IBM®Copyright_and_Trademarks_1 Netfinity®Copyright_and_Trademarks_1 EXP15.

# # $ + K Selecting a RAID Level and Performance Tuning

When selecting a RAID level for your system, consider the following:

|  | Data Redundancy | Hard Disk Drive Capacity Utilization | Read Performance | Write Performance | Built-In Spare Drive |
| --- | --- | --- | --- | --- | --- |
| RAID level-0RAID_Level_0>main | No | 100% | Excellent | Excellent | No |
| RAID level-1RAID_Level_1>main | Yes | 50% | Very Good | Very Good | No |
| RAID level-1 EnhancedRAID_Enhanced_Level_1>main | Yes | 50% | Very Good | Very Good | No |
| RAID level-5RAID_Level_5>main | Yes | 67% to 94% * | Excellent | Good | No |
| RAID level-5 EnhancedRAID_Level_5E>main | Yes | 50% to 88% * | Excellent | Good | Yes |

* Depending on the number of hard disk drives in the array.

# # $ + K Basic Steps in Configuring a ServeRAID Controller

- Select a controller   from the tree view that you want to configure.

Tip: You can select only controllers, but you can expand and collapse the entire tree.

- Select either the Express or the Custom configuration mode; then, click on Next.

| If you select.... |  |
| --- | --- |
| Express mode, the IBM ServeRAID Configuration Program automatically: | Custom mode, the IBM ServeRAID Configuration Program walks you through: |
| - Creates one or more disk arrays, based on the number and size of Ready physical drives in your system. - If four or more Ready physical drives of the same size are available, defines a hot-spare drive. - Defines a logical drive for each array. | - Selecting the physical drives you want to include in your disk arrays. - Selecting the physical drives you want to create as hot-spares. - Defining the logical drives for your arrays. |

- After you review the Configuration Summary, select Apply and the IBM ServeRAID Configuration Program will configure the controller and initialize and synchronize the logical drives.

- When you have completed the ServeRAID controller configuration, exit the IBM ServeRAID Configuration Progam.

- Partition your hard disk drives.

- Install your operating system.

# About_the_IBM_ServeRAID_Configuration_Program

$ About the IBM ServeRAID Configuration Program

K Controller Support;About the IBM ServeRAID Configuration Program;RAID

@ Status|0|0|0|524288||||||

# Current_Status_Screen_Pi

$ System Information Screen

K Logical Drives;synchronization;logical drive:synchronization;system:system information screen

@ Status|0|0|0|524288||||||

# Current_Status_Details_View

$ Using the System Information Screen

K Using;Using:System Information Screen;system:system information screen

@ Status|0|0|0|524288||||||

# Understanding_Tree_Component_Icons

$ Understanding System Information Icons

K Understanding:System Information icons;system:system information

@ Status|0|0|0|524288||||||

# Configure_Adapter_Screen

$ Configure the ServeRAID Controller Screen

K RAID Levels;Logical Drives;Hot Spare;Controller Support;Ready drives;Configure:ServeRAID Controller Screen;Configure;Configure:controller

@ Status|0|0|0|524288||||||

# Express_Configuration_Examples

$ Express Configuration Example

K Logical Drives;Express Configuration Example;Hot Spare;Ready drives

@ Status|0|0|0|524288||||||

# Configure_Adapter_Detail_View

$ Using the Configure Controller Screen

K Using;Logical Drives;Using:Configure Controller Screen;arrays:creating

@ Status|0|0|0|524288||||||

# Custom_Configuration_Screen_B

$ Custom Configuration Physical Drive Screen

K Custom Configuration:physical drive screen

@ Status|0|0|0|524288||||||

# Custom_Configuration_Detail_Tree_View

$ Using the Custom Configuration Physical Drive Screen

K Using;Add New Array;Using:Custom Configuration Physical Drive Screen;Ready;Custom Configuration:physical drive screen;Hot Spare:drives;arrays:creating

@ Status|0|0|0|524288||||||

# Understanding_Custom_Configuration_Physical_Drive_Icons

$ Understanding Custom Configuration Physical Drive Icons

K icons:online drive;Online Drive;icons:physical drives;controller;icons:array;icons;icons:online;SCSI Channel;Ready drives;icons:ready drives;Physical Drives;icons:new hot-spare drive;icons:SCSI drives;icons:ready;icons:controller;icons:new array;Hot Spare:drives;Understanding:Custom Configuration Physical Drive Icons;arrays:creating

@ Status|0|0|0|524288||||||

# Using_this_screen

$ Notes about the Custom Configuration Physical Drive Screen

K Notes about the Custom Configuration Physical Drive Screen;Actions menu;Stripe-Unit Size;Configure:controller;Custom Configuration:physical drive screen

@ Status|0|0|0|524288||||||

# Custom_Configuration_Logical_Drive_Screen_C

$ Custom Configuration Logical Drive Screen

K RAID Levels;Logical Drives;Controller Support;ServeRAID;Custom Configuration:logical drive screen

@ Status|0|0|0|524288||||||

# Custom_Configuration_Logical_Drive_Details_View

$ Using the Custom Configuration Logical Drive Screen

K RAID Levels;Using;Logical Drives;Using:Custom Configuration Logical Drive Screen;Custom Configuration:logical drive screen

@ Status|0|0|0|524288||||||

# More_About_Selecting_the_RAID_Level

$ More About Selecting the RAID Level

K LDM;RAID Levels;Logical Drives;More About Selecting the RAID Level;Change;RAID;RAID:Level-5E

@ Status|0|0|0|524288||||||

# More_About_Selecting_the_Logical_Drive_Size

$ More About Selecting the Logical Drive Size

K RAID Levels;Logical Drives;Adding:logical drives;More About Selecting the Logical Drive Size;Parity;Adding

@ Status|0|0|0|524288||||||

# Understanding_Custom_Configuration_Logical_Drive_Icons

$ Understanding Custom Configuration Logical Drive Icons

K icons:new logical drive;Logical Drives;controller;icons;icons:arrays (plural);icons:controller;icons:new array;Understanding:Custom Configuration Logical Drive Icons

@ Status|0|0|0|524288||||||

# Logical_Drive_Examples

$ Logical Drive Examples

K Logical Drives;Parity;logical drive:examples

@ Status|0|0|0|524288||||||

# Express_Configuration_Screen_D

$ Configuration Summary Screen

K Configuration Summary Screen

@ Status|0|0|0|524288||||||

# Express_Configuration_Summary_View

$ Using the Summary Screen

K Using;Logical Drives;synchronization:start;synchronization;Using:Summary Screen;start:synchronization;start

@ Status|0|0|0|524288||||||

# About_Notes_and_Attentions

$ Notes and Attentions

K Attentions;capacity;Error Messages;capacity:unusable;Unusable Capacity;Notes and Attentions

@ Status|0|0|0|524288||||||

# Event_Viewer

$ Event Viewer

K Using;Using:Event Viewer

@ Status|0|0|0|524288||||||

# DLG_Configure_High_Availability

$ Configure Clustering for ServeRAID

K Cluster:configuring;Cluster;Configure:clustering;Configure:clustering for ServeRAID

@ Status|0|0|0|524288||||||

# IDH_PB_Back

# IDH_PB_Next

# IDH_PB_Cancel

# IDH_AB_File

# IDH_AB_View

# IDH_AB_Actions

# IDH_AB_Help

# IDH_Save_Event_Viewer

# IDH_Clear_Event_Log

# IDH_Exit

# IDH_Select_Large_Icons

# IDH_Select_Small_Icons

# IDH_Options

# IDH_Configure_Adapter

# IDH_Initialize_Configuration

# IDH_EnableDisable_Cache_ReadAhead_Mode

# IDH_Scan_for_NewRemoved_Drives

# IDH_Change_Rebuild_Rate

# IDH_Copy_Configuration_from_Drives_to_Adapters

# IDH_Change_Stripe_Unit_Size

# IDH_Create_Logical_Drive_in_Array

# IDH_Configure_High_Availability

# IDH_Create_Array

# IDH_Delete_All_Arrays

# IDH_Delete_Array

# IDH_Delete_Logical_Drive

# IDH_Change_Write_Cache_Mode

# IDH_Unblock

# IDH_Create_Hot_Spare

# IDH_Remove_Hot_Spare

# IDH_Remove_Defunct_Hot_Spare

# IDH_Replace_Defunct_Hot_Spare

# IDH_Rebuild

# IDH_Set_Drive_to_Online

# IDH_Change_SCSI_Transfer_Speed

# IDH_PB_Apply

# IDH_PB_Modify_Physical_Drives

# IDH_PB_Modify_Logical_Drives

# IDH_PB_Ok

# IDH_Progress_Indicator

# IDH_About

# IDH_Help_Toolbar_icon

# IDH_MSG_1

# IDH_MSG_2

# IDH_MSG_3

# IDH_MSG_4

# IDH_MSG_5

# IDH_MSG_6

# IDH_MSG_7

# IDH_MSG_8

# IDH_MSG_9

# IDH_MSG_10

# IDH_MSG_11

# IDH_MSG_12

# IDH_MSG_13

# IDH_MSG_14

# IDH_MSG_15

# IDH_MSG_16

# IDH_MSG_17

# IDH_MSG_18

# IDH_PB_Start

# IDH_PB_Done

# IDH_Event_View

# IDH_Note_Tab

# IDH_Tab_Add

# IDH_RAID_Level

# IDH_Space_Data

# IDH_PB_Delete_Logical_Drives

# IDH_Percentage_Indicator

# IDH_WT_Current_Status_Tree

# IDH_WT_Current_Status_detail

# IDH_WT_Configure_Adapter_tree

# IDH_WT_Configure_Adapter_details

# IDH_WT_Custom_Configuration_tree

# IDH_WT_Custom_Configuration_details

# IDH_WT_Custom_Config_Logical_tree

# IDH_WT_Custom_Config_Logical_details

# IDH_WT_Express_Config_tree

# IDH_WT_Express_Config_details

# IDH_PB_Apply2

# IDH_DLG_Config_Names

# IDH_DLG_Initiator_IDs

# IDH_DLG_Merge_Group_Info

# Interleave_Depth_and_Stripe_Unit_Size

$ Interleave Depth and Stripe-Unit Size

K How to performance tune your system;Performance Tuning;Stripe-Unit Size and Interleave Depth;Read-Ahead Cache Mode;data-distribution;performance:transaction-based;Stripe-Unit Size;performance:tuning;performance:input/output;performance;transaction-based performance;performance:I/O;I/O requests;stripe:interleave depth;How to performance tune your system:performance enhancements

@ Status|0|0|0|524288||||||

# RAID_Level_0

$ RAID Level-0

K RAID Levels;Logical Drives;Parity;RAID:Level-0;RAID:Level-5;performance:RAID level

A RAID levels

@ Status|0|0|0|524288||||||

# RAID_Level_1

$ RAID Level-1

K RAID Levels;Logical Drives;Mirrored Stripe;stripe:mirrored;RAID:Level-1;Parity;performance:RAID level

A RAID levels

@ Status|0|0|0|524288||||||

# RAID_Enhanced_Level_1

$ RAID Level-1E

K RAID Levels;Enhanced;Logical Drives;Stripe-Unit Size;Data Stripe;Mirrored Stripe;Enhanced:RAID levels;stripe:mirrored;stripe:data;RAID:Enhanced Level-1;RAID:Level 1E;RAID:Level-1E;RAID:Level-1;Parity;performance:RAID level

A RAID levels

@ Status|0|0|0|524288||||||

# RAID_Level_5

$ RAID Level-5

K RAID Levels;performance:RAID level;Logical Drives;performance;Parity;RAID:Level-1;RAID:Level-5

A RAID levels

@ Status|0|0|0|524288||||||

# RAID_Level_5E

$ RAID Level-5E

+ RAIDMAN:0

K RAID Levels;RAID:Level-5E;Enhanced;Enhanced:RAID levels;Parity;performance:RAID level

A RAID levels

@ Status|0|0|0|524288||||||

# Hard_Disk_Drives_Capacities

$ Hard Disk Drives Capacities

K capacity;Hard Disk Drives Capacities;capacity:hard disk drive;arrays:creating

@ Status|0|0|0|524288||||||

# GB

$ GB

K capacity;capacity:hard disk drive;GB

@ Status|0|0|0|524288||||||

# MB

$ MB

K capacity;capacity:usable;MB;capacity:hard disk drive

@ Status|0|0|0|524288||||||

# Logical_Drives

$ Logical Drives

K Logical Drives;Controller Support

@ Status|0|0|0|524288||||||

# Defunct_Drive_glossary

$ Defunct

# Hot_Spare_Drive_glossary

$ Hot-Spare

# Ready_Drive

$ Ready

K Ready drives

@ Status|0|0|0|524288||||||

# Online_Drive

$ Online

K Online;drives:online;drives

@ Status|0|0|0|524288||||||

# Standby_Drive

$ Standby

K Standby:hard disk drive;Standby;Standby:tape;Standby:removeable media;Standby:Timpani (change to actual product name);Standby:CD-ROM;Standby:enclosure

@ Status|0|0|0|524288||||||

# Rebuild_glossary

$ Rebuilding

K drives:rebuilding;Rebuilding

@ Status|0|0|0|524288||||||

# Critical_Migrating_Logical_Drive

$ Critical Migrating Logical Drive

K state:critical;state;Critical:migrating logical drive;Critical:state

@ Status|0|0|0|524288||||||

# Critical_System_Logical_Drive

$ Critical System Logical Drive

K state:critical;state;Critical:state;Critical:system logical drive

@ Status|0|0|0|524288||||||

# Critical_Logical_Drive

$ Critical Logical Drive

K drives:logical;drives:physical;drives;Physical Drives;state:critical;drives:defunct;state;Critical:logical drive;Critical:state;defunct:drives;RAID:Level-1;RAID:Level-5

@ Status|0|0|0|524288||||||

# Migrating_Logical_Drive

$ Migrating Logical Drive

K Logical Drives;drives:logical;Migrating Logical Drive;drives;state:non-critical;Logical Drives:migrating;state

@ Status|0|0|0|524288||||||

# Offline_Logical_Drive

$ Offline Logical Drive

K Offline Logical Drive;Logical Drives;Physical Drives:defunct;drives:physical;drives:logical;defunct:physical drives;drives;defunct:logical drives;Physical Drives;defunct;drives:defunct;Logical Drives:defunct;Logical Drives:offline;defunct:drives;RAID:Level-0;RAID:Level-1;RAID:Level-5

@ Status|0|0|0|524288||||||

# Okay_Logical_Drive

$ Okay Logical Drive

K drives:okay logical drive;drives;Okay Logical Drive

@ Status|0|0|0|524288||||||

# System_Logical_Drive

$ System Logical Drive

K LDM;Logical Drives;drives:logical;drives;Logical Drives:migrating;state:system logical drive;state;system:logical drive

@ Status|0|0|0|524288||||||

# Stripe_Unit_Size

$ Stripe-Unit Size

K Stripe-Unit Size;performance:transaction-based;Logical Drives;How to performance tune your system;I/O;Tuning;performance:tuning;drives:physical;drives:logical;Performance Tuning;Stripe-Unit Size:physical drives, maximum number;How to performance tune your system:performance enhancements;drives;performance:input/output;Physical Drives;transaction-based performance;performance;Stripe-Unit Size:default setting;performance:I/O;I/O requests;stripe:unit size;RAID:Level-5E

@ Status|0|0|0|524288||||||

# Read_Ahead_Cache_Mode

$ Adaptive Read-Ahead Cache Mode

K cache:adaptive;transaction-based performance:sequential;performance:transaction-based;Logical Drives;How to performance tune your system;Stripe-Unit Size;Read-Ahead:Read-Ahead Cache Mode;Read-Ahead:Battery Backup Cache;Read-Ahead:Adaptive Read-Ahead;performance:random;I/O;Tuning;performance:tuning;drives:logical;Performance Tuning;performance:sequential;I/O requests:system;How to performance tune your system:performance enhancements;drives;cache:read-ahead;Read-Ahead;performance:input/output;transaction-based performance:steady;transaction-based performance;performance;cache;transaction-based performance:random;Read-Ahead Cache Mode;performance:I/O;I/O requests

@ Status|0|0|0|524288||||||

# Tree_View_Icons

$ Understanding the Tree View Icons

K icons:online drive;icons:physical drives;icons:logical drives;icons:error, controller;icons:error;icons:array;icons;Error;icons:hot-spare drives (plural);icons:CD-ROM drive;icons:logical drives (plural);icons:enclosure;icons:alert;CD-ROM Drive;icons:alert, controller;icons:removeable media drive;icons:ready drives;icons:arrays (plural);Removable Media Drive;icons:tape drive;icons:hot-spare drive;icons:SCSI channel;icons:controller;Understanding:Tree view icons;icons:physical drives (plural)

@ Status|0|0|0|524288||||||

# Event_Log_Icons

$ Event Log Icons

K Using;icons:error;icons;Event Viewer;Using:Event Viewer;icons:alert;icons:information;Event Log Icons

@ Status|0|0|0|524288||||||

# Server_System_Information

$ Server System Information Field Definitions

K information field definitions:server system;Server System Information Field Definitions;information field definitions

A Field Definitions

@ Status|0|0|0|524288||||||

# Adapter_System_Information

$ Controller System Information Field Definitions

K controller;Stripe-Unit Size;Data Scrubbing;Battery Backup Write Cache;Unattended Mode;information field definitions:controller system;Auto-Synchronization;Controller System Information Field Definitions;information field definitions

A Field Definitions

@ Status|0|0|0|524288||||||

# Array_Information

$ Arrays Information

# Logical_Drive_Information

$ Logical Drives Information Field Definitions

K Logical Drives Information Field Definitions

A Field Definitions

# Physical_Drives_Information

$ Physical Drives Information

# Array_Query_Information

$ Array Query Information Field Definitions

K information field definitions:array query;Array Query Information Field Definitions;information field definitions

A Field Definitions

@ Status|0|0|0|524288||||||

# Hot_Spare_Drive_Information

$ Hot-Spare Drives Information

K Hot Spare:drives information field definition

# Logical_Drives_in_Array_Query_Information

$ Logical Drives in Array Query Information Field Definitions

K Logical Drives;Logical Drives in Array Query Information Field Definitions;information field definitions:logical drives in array query;Logical Drives:in array query;information field definitions

A Field Definitions

@ Status|0|0|0|524288||||||

# Physical_Drives_in_Arrays_Query_Information

$ Physical Drives in Array Query Information

# Logical_Drive_Query_Information

$ Logical Drive Query Information Field Definitions

K logical drive:query information field definitions

A Field Definitions

# Physical_Drive_Information

$ Physical Drive Information Field Definitions

K Physical Drive Information Field Definitions

A Field Definitions

# Channel_Information

$ Channel Information Field Definitions

K Channel Information Field Definitions

A Field Definitions

# Cluster

$ Cluster

K Cluster;High-Availability;Merge or Unmerge Logical Drives

# Battery_Backup_Cache

$ Battery Backup Cache

K Battery Backup Cache;cache:battery backup;cache

@ Status|0|0|0|524288||||||

# Hot_Swap_Rebuild

$ Hot-Swap Rebuild

K drives:physical;rebuild;Rebuilding;drives;defunct:physical drives;defunct:hot-swap;defunct:drives;rebuild:hot-swap;Hot-Swap Rebuild;defunct;drives:defunct

@ Status|0|0|0|524288||||||

# Auto_Synchronization

$ Auto-Synchronization

K RAID:Level-0;RAID:Level-1;synchronization;logical drive;RAID:Level-5;synchronization:automatic;RAID;Auto-Synchronization;Synchronize Drives

@ Status|0|0|0|524288||||||

# Data_Scrubbing

$ Data Scrubbing

K logical drive:data scrubbing;logical drive:synchronization;Data Scrubbing;synchronization:data scrubbing;synchronization;logical drive;synchronization:automatic;Auto-Synchronization:data scrubbing;Auto-Synchronization;Synchronize Drives

@ Status|0|0|0|524288||||||

# PFA_Error

$ PFA Error

K drives:rebuilding;Physical Drives:defunct;drives:physical;drives;defunct:physical drives;Physical Drives;defunct:drives;Hot-Swap Rebuild;defunct;drives:defunct;PFA Error

@ Status|0|0|0|524288||||||

# Blocked_Access

$ Blocked Logical Drive

K RAID:Level-0;Logical Drives;RAID:Level-1;logical drive:blocked;rebuild;corrupt data;logical drive;Rebuilding;RAID:Level-5;state:blocked;Blocked Logical Drive;data:reconstruction;data;Logical Drives:blocked;state;Logical Drives:defunct;data:corruption

@ Status|0|0|0|524288||||||

# Write_Cache_Mode

$ Write-Cache Mode

K Performance Tuning;Write-Cache Mode;Write Through;transaction-based performance;performance;performance:hard disk drive, writing to;cache;Write Back

@ Status|0|0|0|524288||||||

# Write_Back

$ Write Back

K How to performance tune your system;Read-Ahead:Battery Backup Cache;Tuning;performance:tuning;cache:battery backup;Performance Tuning;Write-Cache Mode;How to performance tune your system:performance enhancements;Write Through;performance;performance:hard disk drive, writing to;cache;Write Back

@ Status|0|0|0|524288||||||

# Write_Through

$ Write Through

K How to performance tune your system;Read-Ahead:Battery Backup Cache;Tuning;performance:tuning;cache:battery backup;Performance Tuning;Write-Cache Mode;How to performance tune your system:performance enhancements;Write Through;performance;performance:hard disk drive, writing to;Write Back

@ Status|0|0|0|524288||||||

# Number_ServeRAID_controllers

$ Could not determine the number of ServeRAID controllers.

K controller;Error Messages;Could not...

@ Status|0|0|0|524288||||||

# Apply_configuration_ServeRAID

$ Could not apply the complete configuration to ServeRAID...

K Error Messages;Could not...;Configure:failing to;Configure

@ Status|0|0|0|524288||||||

# Not_create_logdrive

$ Could not create the new logical drive on ServeRAID...

K Error Messages;Could not...;logical drive;logical drive:failure to create new

@ Status|0|0|0|524288||||||

# Synchronize_Drives

$ Synchronize Drives

K View menu;logical drive:synchronization;synchronization;RAID:Level-5;View menu:Options dialog;RAID;data;Synchronize Drives;Options dialog;data:redundancy;RAID:Level-5E

@ Status|0|0|0|524288||||||

# ViewOptions

$ View Options

K mode:synchronization;Initialization Mode;View Options;mode;logical drive:initialization;mode:initialization;logical drive:synchronization;RAID:Level-5;View menu:Options dialog;Auto-Synchronization;Synchronization Mode

@ Status|0|0|0|524288||||||

# IDH_Init_Logical_Drive

# IDH_Synch_Logical_Drive

# IDH_PB_OK_Ack

# Finding_Information_on_the_World_Wide_Web

$ Finding Information on the World Wide Web

K Adapter Support;Finding Information on the World Wide Web;Controller Support;Server Information;Configure:clustering for ServeRAID

A Adapter Support;Configure Clustering for ServeRAID;Controller Support;Server information

# Enclosure

$ Enclosure

# Free_Space

$ Free Space

K arrays:creating;free space

# Configuration_Mode

$ Configuration Mode

# Information_Mode

$ Information Mode

# IDH_Sure_synchronize_Logical_Drive

# How_to_performance_tune_your_system

$ How to Performance Tune Your System

+ RAIDMAN:0

K How to performance tune your system:parameters;How to performance tune your system;Tuning;Performance Tuning;How to performance tune your system:performance enhancements;performance;Stripe-Unit Size and Interleave Depth;Read-Ahead Cache Mode;More About Selecting the RAID Level

A Adaptive Read-Ahead Cache Mode;Interleave Depth and Stripe Size;Parameters;Performance Enhancements;Read-Ahead Cache Mode;Tuning

@ Status|0|0|0|524288||||||

# Application_Environment_Groups_Reference

$ Application Environment Groups Reference

+ RAIDMAN:0

K Application Environment Groups Reference;Performance Tuning;I/O requests

# Critical_State

$ Critical State

+ RAIDMAN:0

K Critical:state;state:critical

@ Status|0|0|0|524288||||||

# High_Availability

$ High-Availability

+ RAIDMAN:0

K High-Availability;Cluster

# Parity

$ Parity

+ RAIDMAN:0

K Parity;RAID;RAID:Level-5E

# Total_Disk_Space

$ Total Disk Space

+ RAIDMAN:0

K Total Disk Space;arrays:creating;Hard Disk Drives Capacities

A Unusable Capacity

# Unusable_Capacity

$ Unusable Capacity

+ RAIDMAN:0

K Unusable Capacity;arrays:creating;Hard Disk Drives Capacities

A Total Disk Space

# Usable_Capacity

$ Usable Capacity

+ RAIDMAN:0

K Usable Capacity;arrays:creating;Hard Disk Drives Capacities

# Hot_Spare

$ Hot Spare

+ RAIDMAN:0

K Hot Spare

# Adaptive_Read_Ahead_Mode

$ Adaptive Read-Ahead Mode

+ RAIDMAN:0

K Adaptive Read-Ahead Mode;How to performance tune your system;Battery Backup Cache;Performance Tuning;Read-Ahead Cache Mode;Tuning;How to performance tune your system:performance enhancements

A Parameters;Performance Enhancements;Read-Ahead Cache Mode;Tuning

# IDH_DLG_Identify_Drives

$ Identify Drives

+ RAIDMAN:0

K Identify Drives

A Installing a Drive

# Restore_to_Factory_Defaults

$ Restore to Factory Defaults

+ RAIDMAN:0

K Restore to Factory Defaults;Delete All Arrays

A Controller Support;Delete All Arrays;Factory Defaults;Parameters;Saving to diskette

# IDH_Identify_Drives

K SAF-TE

# Unattended_Mode

$ Unattended Mode

+ RAIDMAN:0

K Unattended Mode;Cluster;Merge or Unmerge Logical Drives

# IDH_Unattended_Mode

# IDH_Merge_Unmerge_Logical_Drives

# Merge_Unmerge_Logical_Drives

$ Merge / Unmerge Logical Drives

+ RAIDMAN:0

K Merge or Unmerge Logical Drives;Cluster

# What_s_New

$ What's New in the ServeRAID Configuration Utility

+ RAIDMAN:0

K Cluster:configuring;cache:adaptive;Cluster;Read-Ahead:Adaptive Read-Ahead;Read-Ahead;RAID:Level-5E;What's New in the ServeRAID Configuration Utility;Adaptive Read-Ahead Mode

# Copyright_and_Trademarks_1

$ Copyright

K Copyright;Trademarks

# SAF_TE

$ SAF-TE

+ RAIDMAN:0

K SAF-TE

# Selecting_a_RAID_Level

$ Selecting a RAID Level

+ RAIDMAN:0

K Selecting a RAID Level;Performance Tuning;RAID;RAID:Level-5E;How to performance tune your system:performance enhancements;How to performance tune your system;More About Selecting the RAID Level;Parity;performance

# Basic_Steps_in_Configuring_a_ServeRAID_Controller

$ Basic Steps in Configuring a ServeRAID Controller

+ RAIDMAN:0

K Server;Configuring;Order of steps;Configuring:controller;Steps:Overall;Steps;Server:configuring;System:configuring;Configuring:system;Controller:configuring;Steps:Order;Steps:Basic;Configuring:server;Basic Steps in Configuring a ServeRAID Controller;overall steps;Controller;System